# NUC-G BIOS ENGINEERING

Intel Corporation, 5200 NE Elam Young Parkway
	Hillsboro, OR  97124-6497

BIOS Update Release Notes

Tiger Lake-U Standard BIOS

PRODUCTS: NUC11TNBi3/K/H, NUC11TNBi5/K/H, NUC11TNBi7/K/H (Tiger Canyon)

### TNTGL357.0071.2023.0130.1120 Production BIOS

About This Release:

- Date: 01/30/2023

- ROM Image Checksum: 1065

- EC Firmware: TIGA400000

- EC Firmware TNL: TIGA400A00

- ME Firmware: 15.0.35.2039

- PCH Configuration Firmware: pchc_tgl_15.0.0.1021_prod

- PMC Firmware: TGPLP_B0_PMC_FW_150.01.20.1039_prod

- iTBT Firmware: TBT_TGLREV_46_prod

- IOM Firmware: iomp_11.0016.0.0_prod

- NPHY Firmware: PHY_B0_11.225.256.2041_prod

- CRB Label: 049

- Boot Guard ACM: v1.14.25

- Memory Reference Code: Based on 0A.00.66.10

- Integrated Graphics:

  - UEFI Driver: 17.0.1073

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.1.1.5201

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: E0903X7

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_000000A6

New Fixes/Features:

Based on TN0070

- Fixed: [EIP702484][PTK2712-CRB/OEM] UEFI Variable access vulnerability in Intel NUC BIOS.

- Fixed: [EIP724087][SA50170] SmmEntryPoint Underflow Vulnerability.

- Fixed: [EIP724403][SA50157] UEFI Boot Variables Access.

- Fixed: [EIP725307][SA50148] SDIO_DEV_CONFIGURATION SetVariable NVRAM Corruption.

- Fixed: [EIP723671][PTK2975] OS Kernel-level malware may cause information disclosure vulnerability.

- Fixed: [EIP711616][PTK2696] Intel NUC information disclosure vulnerability.

- Fixed: [EIP701942][SA50085] GRUB Bootloader Vulnerability.

- Fixed: [EIP722927] F4 BIOS Recovery can’t read BIOS CAP file from USB device using the Type-C USB Port.

- Fixed: [EIP725431] IO Crest's M.2 LAN module fails to be recognized in the system.

Known Errata:

- Blocked BIOS Flash downgrade to TN0070 and previous versions due to a change of NVRAM settings.

- TN0063 updating ME FW to 15.0.35.x, due to ME FW limitation, F7 flash to TN0062 or previous release cannot downgrade ME FW.

- Blocked BIOS flash downgrade to TN0063 (Non-vPro) and TNv0064 (vPro) due to CRB update for Win11 support.

- EIP530307, Intel comments that SAGV does not function at this moment, therefore it is disabled in current TN BIOS.

- WU files for test use only.

Tiger Lake-U Standard BIOS

PRODUCTS: NUC11TNBi3/K/H, NUC11TNBi5/K/H, NUC11TNBi7/K/H (Tiger Canyon)

### TNTGL357.0070.2022.1028.1209 Production BIOS

About This Release:

- Date: 10/28/2022

- ROM Image Checksum: 329f

- EC Firmware: TIGA400000

- EC Firmware TNL: TIGA400A00

- ME Firmware: 15.0.35.2039

- PCH Configuration Firmware: pchc_tgl_15.0.0.1021_prod

- PMC Firmware: TGPLP_B0_PMC_FW_150.01.20.1039_prod

- iTBT Firmware: TBT_TGLREV_46_prod

- IOM Firmware: iomp_11.0016.0.0_prod

- NPHY Firmware: PHY_B0_11.225.256.2041_prod

- CRB Label: 049

- Boot Guard ACM: v1.14.25

- Memory Reference Code: Based on 0A.00.66.10

- Integrated Graphics:

  - UEFI Driver: 17.0.1073

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.1.1.5201

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: E09.03x7

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX	MX25L25645G	32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_000000A6

New Fixes/Features:

Based on TN0069

- Fixed: [EIP694936][PTK2617] SMM memory corruption vulnerability in SMM driver on Intel platforms.

- Added: [EIP695602][SA50120] BootPerformanceTable_pointer.

- Updated: ME FW to 15.0.35.2039.

- Updated: CPU Microcode to M80806C1_000000A6.

- Fixed: [EIP697888][PTK2778] Stack overflow vulnerability in SMI handler.

- Fixed: [EIP701856][PTK2911/SA50133] RSB Stuffing Mitigation for Speculative Execution Vulnerability.

- Fixed: [EIP701942][SA50085] GRUB Bootloader Vulnerability.

- Updated: [EIP707101][IPU] 2022 IPU update: 2022.1/2022.2/2022.3.

- Fixed: [EIP707143][PTK2712/SA50151] SIO_DEV_STATUS_VAR_NAME Information Leakage_PTK2712#12.

- Fixed: [EIP707901][PTK2872] Potential hack of BIOS EBU Flash DLL.

- Fixed: [EIP710985][SA50088] TianoCore Security Issues.

- Added: [EIP711019] Implement S3 reboot code in MS support project.

- Fixed: [EIP711083][PTK2828/PTK2829] Intel NUC 8 vulnerability/Intel NUC 8 info leak vulnerability.

- Fixed: [EIP711499] TN vPro NUC is unable to detect the Thunderbolt connection with Mac. (ME FW 15.0.41.2142).

- Added: StdDefaults into ProtectedNvVariableForRuntime eLink.

- Added: VR current limit calculation bug fix.

- Fixed: [EIP701727] Network Boot option IPv4 & IPv6 disappeared, after setting Network Boot to UEFI PXE & iSCSI.

- Fixed: [EIP705967] UQI Value SPG12B is duplicated in BIOS 0067 and 0069.

- Fixed: [EIP709354] BIOS Setup item map string is incorrect.

Known Errata:

- TN0063 updating ME FW to 15.0.35.x, due to ME FW limitation, F7 flash to TN0062 or previous release cannot downgrade ME FW.

- Blocked BIOS flash downgrade to TN0063 (Non-vPro) and TNv0064 (vPro) due to CRB update for Win11 support.

- EIP530307, With Intel comments that SAGV does not function at this moment, therefore it is disabled in current TN BIOS.

- WU files for test use only.

Tiger Lake-U Standard BIOS

PRODUCTS: NUC11TNBi3/K/H, NUC11TNBi5/K/H, NUC11TNBi7/K/H (Tiger Canyon)

### TNTGL357.0069.2022.0816.1135 Production BIOS

About This Release:

- Date: Aug, 16 2022

- ROM Image Checksum: 61a4

- EC Firmware: TIGA400000

- EC Firmware TNL: TIGA400A00

- ME Firmware: 15.0.35.1898v3

- PCH Configuration Firmware: pchc_tgl_15.0.0.1021_prod

- PMC Firmware: TGPLP_B0_PMC_FW_150.01.20.1039_prod

- iTBT Firmware: TBT_TGLREV_43_prod

- IOM Firmware: iomp_11.0015.0.0_prod

- NPHY Firmware: PHY_B0_11.225.256.2041_prod

- CRB Label: 049

- Boot Guard ACM: v1.14.25

- Memory Reference Code: Based on 0A.00.66.10

- Integrated Graphics:

  - UEFI Driver: 17.0.1073

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.1.1.5201

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.9.03

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - m80806c1_0000009A

New Fixes/Features:

- Fixed: [EIP688660][PTK2702] Arbitrary write vulnerability in PEI module leads to arbitrary code execution in PEI phase.

- Fixed: [EIP689874][PTK2699] Information disclosure vulnerability.

- Fixed: [EIP690960][PTK2793] Intel NUC information leak vulnerability.

- Fixed: [EIP694458][PTK2703] Arbitrary code execution in DXE driver.

- Fixed: [EIP694645][PTK1931] Privilege escalation vulnerability from kernel to SMM in multiple devices.

- Updated: Updated FW for PCON FW RTD2175_PEGATRON_TIGER_CANYON_User_V2p30.bin.

- Added: [EIP641882] About screen when we do the WU update.

- Added: [EIP678219] Layout change for the F6 Manufacturing Mode exit string.

- Added: [EIP678220] Message warning displayed when the current BIOS upgrade means a downgrade is not possible.

- Fixed: AutoMeudLib minor version check error.

- Fixed: [EIP680611] iSetupCfg tool to modify power limit value, not getting changed.

- Fixed: [EIP690129] POST Hotkey message does not display when Secure Boot is Enabled.

- Fixed: [EIP698163] Downgrading from BIOS 0064 to 0062 bricking TN NUCs.

Known Errata:

- TN0063 updating ME FW to 15.0.35.x, due to ME FW limitation, F7 flash to TN0062 or previous release cannot downgrade ME FW.

- Blocked BIOS downgrade to TN0063 (Non-vPro) and TNv0064 (vPro) due to CRB update for Win11 support.

- EIP530307, With Intel comments that SAGV does not function at this moment, therefore it is disabled in current TN BIOS.

- WU files for test use only.

Tiger Lake-U Standard BIOS

PRODUCTS: NUC11TNBi3/K/H, NUC11TNBi5/K/H, NUC11TNBi7/K/H (Tiger Canyon)

### TNTGL357.0067.2022.0718.1742 Production BIOS

About This Release:

- Date: Jul, 18 2022

- ROM Image Checksum: c165

- ME Firmware: 15.0.35.1898v3

- EC Firmware: TIGA400000

- EC Firmware TNL: TIGA400A00

- PMC Firmware: TGPLP_B0_PMC_FW_150.01.20.1039_prod

- Boot Guard ACM: v1.14.25

- Memory Reference Code: Based on 0A.00.66.10

- Integrated Graphics:

  - UEFI Driver: 17.0.1073

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.1.1.5201

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.9.03

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_0000009A

New Fixes/Features:

- Based on TN0066

- Fixed: [EIP686460] Rally Bar speaker produces static noise after upgrading to BIOS 0064.

- Updated: [BIOS 0066] GOP FW to 17.0.1073.

Known Errata:

- TN0063 updating ME FW to 15.0.35.x, due to ME FW limitation, F7 flash to TN0062 or previous release cannot downgrade ME FW.

- TN0032 EIP560766 Added code change for new DmiEdit tool, please use new tools.

- EIP530307, With Intel comments that SAGV does not function at this moment, therefore it is disabled in current TN BIOS.

- WU files for test use only.

### TNTGL357.0066.2022.0615.1051 Development BIOS

About This Release:

- Date: Jun, 15 2022

- ROM Image Checksum: e570

- ME Firmware: 15.0.35.1898v3

- EC Firmware: TIGA400000

- EC Firmware TNL: TIGA400A00

- PMC Firmware: TGPLP_B0_PMC_FW_150.01.20.1039_prod

- Boot Guard ACM: v1.14.25

- Memory Reference Code: Based on 0A.00.66.10

- Integrated Graphics:

  - UEFI Driver: 17.0.1073

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.1.1.5201

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.9.03

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - m80806c1_0000009a

New Fixes/Features:

- Based on TN0065

- Fixed: [EIP647062][PTK1885] Unauthorized modification of UEFI variables could disable the protect mechanism of SMM.

- Added: [EIP650023] Press F6 from BIOS Setup to exit MFG mode if enabled.

- Added: [EIP678219] Added Layout for the F6 string from the BIOS Master RDv3.10.01.

- Fixed: [EIP660000] Unit will always reboot if changing HW configuration and enabling TXT after provision.

- Fixed: [EIP663956] System can't boot up after changing settings.

- Fixed: [EIP666086] BIOS patch of boot options overflow handling.

- Updated: [EIP670621] Updated RTD2175 PCON FW.

- Fixed: [EIP672442] Execute command via WMIC can't catch 2nd DDR information.

- Added: [EIP672875] Added BIOS Warning message during BIOS WU.

- Fixed: [EIP675624] Watchdog does not restart NUC under Ubuntu 20.04 (LTS).

- Fixed: [EIP680835][PTK1433] PEI memory corruption on server boards and on majority of NUCs.

- Fixed: [EIP681956] PTT stops working after PCH power loss.

- Fixed: [EIP684243] Fix for system always reset loop after RTC power loss.

- Fixed: [EIP684919][PTK2427] The stack buffer overflow vulnerability leads to arbitrary code execution in DXE driver on Intel platform.

- Fixed: [EIP685671][PTK2425] Buffer overflow in UEFI Firmware BIOS core.

- Fixed: [EIP686150] After disabling all the USB ports in the BIOS, the unit was unrecoverable, even after BIOS recovery.

- Fixed: [EIP682356] The Intel NUC logo size of HDMI port 1 was not the same as the Intel NUC logo size of HDMI port 2 when SUT was connected to 4K monitor.

- Updated: IGD GOP FW to 17.0.1073.

Known Errata:

- TN0063 updating ME FW to 15.0.35.x, due to ME FW limitation, F7 flash to TN0062 or previous release cannot downgrade ME FW.

- TN0032 EIP560766 Added code change for new DmiEdit tool, please use new tools.

- EIP530307, With Intel comments that SAGV does not function at this moment, therefore it is disabled in current TN BIOS.

- WU files for test use only.

### TNTGL357.0065.2022.0426.1016 Development BIOS

About This Release:

- Date: Apr, 26 2022

- ROM Image Checksum: 8f1a

- ME Firmware: 15.0.35.1898v3

- EC Firmware: TIGA400000

- EC Firmware TNL: TIGA400A00

- PMC Firmware: TGPLP_B0_PMC_FW_150.01.20.1039_prod

- Boot Guard ACM: v1.14.25

- Memory Reference Code: Based on 0A.00.66.10

- Integrated Graphics:

  - UEFI Driver: 17.0.1071

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.1.1.5201

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.9.03

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - m80806c1_0000009a

New Fixes/Features:

Based on TN0064

- Updated: CRB0 0049 code.

- Updated: IGD GOP FW to 17.0.1071

Known Errata:

- TN0063 updating ME FW to 15.0.35.x, due to ME FW limitation, F7 flash to TN0062 or previous release cannot downgrade ME FW.

- TN0032 EIP560766 Added code change for new DmiEdit tool, please use new tools.

- EIP530307, With Intel comments that SAGV does not function at this moment, therefore it is disabled in current TN BIOS.

- WU files for test use only.

Tiger Lake-U Standard BIOS

PRODUCTS: NUC11TNBi3/K/H, NUC11TNBi5/K/H, NUC11TNBi7/K/H (Tiger Canyon)

### TNTGL357.0064.2022.0217.1550 Production BIOS

About This Release:

- Date: Feb, 06 2022

- ROM Image Checksum: a5a1

- ME Firmware: 15.0.35.1898v3

- EC Firmware: TIGA40.00.00

- EC Firmware TNL: TIGA40.0A.00

- PMC Firmware: TGPLP_B0_PMC_FW_150.01.20.1039_prod

- Boot Guard ACM: v1.14.25

- Memory Reference Code: Based on 0A.00.5D.32

- Integrated Graphics:

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.1.1.5201

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.9.03

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - m80806c1_0000009a

New Fixes/Features:

- Based on TN0062

- Updated: CRB 048 code for Windows 11 support.

- Updated: EC FW to 40.00.00 / 40.0A.00.

- Updated: ME FW to 15.0.35.1898v3 Consumer for Windows 11 BKC support.

- Fixed: [EIP642974] DBIOS for virtual Display emulation doesn’t allow BIOS KVM access.

Known Errata:

- TN0063 updating ME FW to 15.0.35.x, due to ME FW limitation, F7 flash to TN0062 or before cannot downgrade ME FW.

- TN0032 EIP560766 Added code change for new DmiEdit tool, please use new tools.

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- WU files for test use only.

### TNTGL357.0063.2022.0106.1923 Development BIOS

About This Release:

- Date: Jan, 06 2022

- ROM Image Checksum: 0a1c

- ME Firmware: 15.0.35.1898v3

- EC Firmware: TIGA400000

- EC Firmware TNL: TIGA400A00

- PMC Firmware: TGPLP_B0_PMC_FW_150.01.20.1039_prod

- Boot Guard ACM: v1.14.25

- Memory Reference Code: Based on 0A.00.5D.32

- Integrated Graphics:

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.1.1.5201

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.9.03

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - m80806c1_0000009a

New Fixes/Features:

- Based on TN0062

- Update CRB 048 for Win11

- Update EC FW 400000 / 400A00

- Update ME FW 15.0.35.1898v3 Cons & 15.0.35.1951v6.1 Corp for Win11 BKC

- eip642974 DBIOS for virtual Display emulation doesn’t allow BIOS KVM access

Known Errata:

- TN0063 update ME FW 15.0.35.x, due to ME FW limitation, F7 flash to TN0062 or before cannot downgrade ME FW.

- TN0032 EIP560766 Add code change for new DmiEdit tool, please use new tools

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

Tiger Lake-U Standard BIOS

PRODUCTS: NUC11TNBi3/K/H, NUC11TNBi5/K/H, NUC11TNBi7/K/H (Tiger Canyon)

### TNTGL357.0062.2021.1203.1108 Production BIOS

About This Release:

- Date: Dec, 03 2021

- ROM Image Checksum: 9700b3e6

- ME Firmware: 15.0.10.1574 (v1.2)

- EC Firmware: TIGA39.00.00

- EC Firmware NTBT: TIGA39.00.00NTBT

- PMC Firmware: PMC_B0_150.01.20.1032_prod

- Boot Guard ACM: 1.14.15

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.1.1.5201

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.9.03

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_0000009A

  - M80806C1_0000007E

New Fixes/Features:

- Based on TN0061

- Added: EC FW 390000NTBT for NTBT SKU.

- Fixed in Development BIOS 0060:

- Updated: EC FW to 39.00.00

- Updated: CPU Microcode to M80806C1_0000009A, M80806C1_0000007E.

- Fixed: Weird BIOS Message when Recovering BIOS on certain NUC products (PH).

- Fixed: [eip603248] WOL from Modern Standby doesn't work in Windows 10 20H2 with latest BIOS releases.

- Fixed: [eip603755] Need to update Pre OS driver for VMD support.

- Fixed: [eip606761] PTK1459 SMM arbitrary code execution may allow attacker to modify SPI flash and launch the BIOS bootkit.

- Added: [eip606764] PTK1439 Implement BIOSGuard solution for PEI buffer overflow.

- Fixed: [eip607535] PTK1484 able to achieve arbitrary write in SMRAM save state region.

- Fixed: [eip612616] Hidden non-use BIOS setup item.

- Fixed: [eip613982] PTK1457 Vulnerable EFI variable read procedure.

- Fixed: [eip632901] USB Device Not Recognized with Type-C to HDMI Cable/Dongle (EC FW 390000 fix).

- Added: [eip633781] CEC discussion for Player Failover Utility.

- Fixed: [eip635091] Linux ACPI BIOS error.

- Fixed: [eip640584] BIOS doesn't disable wireless module completely.

- Fixed: [eip641399] WMI HDMI CEC-HDMI port mapping issue (EC FW 390000 fix).

Known Errata:

- Flashing from TN0052/TN0053 to TN0054 still stay on POST logo for about 4mins, TN0054 to TN0054 will not see this.

- TN0032 EIP560766 added code change for new DmiEdit tool, please use latest version of AMI/Intel tools.

- EIP530307, With Intel comment that  SAGV does not function at this moment, therefore disable it in current TN BIOS.

- WU files for test use only.

### TNTGL357.0061.2021.1201.0937 Development BIOS

About This Release:

- Date: Dec, 01 2021

- ROM Image Checksum: 77b9a87c

- ME Firmware: 15.0.10.1574 (v1.2)

- EC Firmware: TIGA39.00.00

- PMC Firmware: PMC_B0_150.01.20.1032_prod

- Boot Guard ACM: 1.14.15

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.5.5115

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - m80806c1_0000009a

  - m80806c1_0000007e

New Fixes/Features:

- Based on TN0060

- Changed GPIO GPP_C12 Non-TBT: GPIO Output High.  TBT: GPIO InputInv output Low, Int=APIC.

Known Errata:

- Flash from TN0052 or TN0053 to TN0054 still stay on POST logo for about 4mins, TN0054 to TN0054 will not wait 4 mins.

- TN0032 EIP560766 Add code change for new DmiEdit tool, please use new tools

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- WU files for test use only.

### TNTGL357.0060.2021.1110.1830 Development BIOS

About This Release:

- Date: Nov, 10 2021

- ROM Image Checksum: 9701F5F9

- ME Firmware: 15.0.10.1574 (v1.2)

- EC Firmware: TIGA39.00.00

- PMC Firmware: PMC_B0_150.01.20.1032_prod

- Boot Guard ACM: 1.14.15

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.5.5115

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - m80806c1_0000009a

  - m80806c1_0000007e

New Fixes/Features:

- Based on TN0059

- Update EC FW 390000

- Updated: CPU Microcode firmware to M80806C1_0000009A, M80806C1_0000007E.

- Weird BIOS Message when Recovering BIOS on PH

- eip603248 WOL from Modern Standby doesn't work in Windows 10 20H2 with latest BIOS releases

- eip603755 Need to update Pre OS driver for VMD support

- eip606761 PTK1459 SMM arbitrary code execution may allow attacker to modify SPI flash and launch the BIOS bootkit

- eip606764 PTK1439 Implement BiosGuard solution for PEI buffer overflow

- eip607535 PTK1484 able to achieve arbitrary write in SMRAM save state region

- eip612616 Hidden non-use BIOS setup item"

- eip613982 PTK1457 Vulnerable EFI variable read procedure

- eip632901 USB Device Not Recognized with Type-C to HDMI Cable/Dongle (EC FW 390000 fix)

- eip633781 CEC discussion for Player Failover Utility

- eip635088 Need AMI release DBIOS for TN remove TBT/Type-C connect SKU. (require EC FW 390000)

- eip635091 Linux ACPI BIOS error

- eip640584 BIOS doesn't disable wireless module completely

- eip641399 WMI HDMI CEC-HDMI port mapping issue (EC FW 390000 fix)

Known Errata:

- Flash from TN0052 or TN0053 to TN0054 still stay on POST logo for about 4mins, TN0054 to TN0054 will not wait 4 mins.

- TN0032 EIP560766 Add code change for new DmiEdit tool, please use new tools

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- WU files for test use only.

Tiger Lake-U Standard BIOS

PRODUCTS: NUC11TNBi3/K/H, NUC11TNBi5/K/H, NUC11TNBi7/K/H (Tiger Canyon)

### TNTGL357.0058.2021.0813.1709 Production BIOS

About This Release:

- Date: Aug, 13 2021

- ROM Image Checksum: 683d13cf

- ME Firmware: 15.0.10.1574 (v1.2)

- EC Firmware: TIGA37.00.00

- PMC Firmware: PMC_B0_150.01.20.1032_prod

- Boot Guard ACM: 1.14.15

- Memory Reference Code: Based on 0A.00.38.51

- Integrated Graphics

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.5.5115

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - m80806c1_0000008a

  - m80806c1_0000007e

New Fixes/Features:

- Based on TN0057

- Updated: EC Firmware updated to TIGA37.00.00

- Updated: PMC Firmware updated to PMC_B0_150.01.20.1032_prod.

- Fixed: EC Firmware text typo during BIOS flash update.

- Updated: CPU Microcode to m80806c1_0000008a, m80806c1_0000007e.

- Updated: ME Firmware package to 15.0.10.1574v1.2

- Added: [EIP601236] To include secure boot PK and related DB into Aptio project source tree.

- Added: [EIP605460] Event log support for Auto RTC feature.

- Fixed: [EIP608543] Virtual Display emulation doesn’t allow BIOS KVM access (updated to EC FW 370000).

- Added: [EIP618160] Put the PCON FW Update FMP driver into the BIOS region.

- Added: [EIP618316] Pop-up warning message to user when transition BIOS is necessary.

- Added: [EIP620606] BIOS New Item: Force-enable power to 2242 m.2 slot.

- Fixed: [EIP626041] Persistent display emulation did not work properly (updated to EC FW 370000).

Known Errata:

- Flashing from TN0052/TN0053 to TN0054 still stay on POST logo for about 4mins, TN0054 to TN0054 will not see this.

- TN0032 EIP560766 added code change for new DmiEdit tool, please use latest version of AMI/Intel tools.

- EIP530307, With Intel comment that  SAGV does not function at this moment, therefore disable it in current TN BIOS.

- WU files for test use only.

Tiger Lake-U Standard BIOS

PRODUCTS: NUC11TNBi3/K/H, NUC11TNBi5/K/H, NUC11TNBi7/K/H (Tiger Canyon)

### TNTGL357.0057.2021.0609.1511 Production BIOS

About This Release:

- Date: Jun, 9 2021

- ROM Image Checksum: 7f54fc8b

- ME Firmware: 15.0.10.1469

- EC Firmware: TIGA360000

- PMC Firmware: 150.01.20.1028

- Boot Guard ACM: 1.14.15

- Memory Reference Code: Based on 0A.00.38.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.5.5115

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_00000072.pdb

New Fixes/Features:

- Based on TN0056

- Fixed: [AMI Changes][EIP617265] MFG Mode was not disabled before shipping.

Known Errata:

- Flashing from TN0052 or TN0053 to TN0054 still stay on POST logo for about 4 mins, TN0054 to TN0054 will not wait 4 mins.

- TN0032 EIP560766 Added code change for new DmiEdit tool, please use new tools

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- WU files for test use only.

Tiger Lake-U Standard BIOS

PRODUCTS: NUC11TNBi3/K/H, NUC11TNBi5/K/H, NUC11TNBi7/K/H (Tiger Canyon)

### TNTGL357.0056.2021.0513.1618 Production BIOS

About This Release:

- Date: May, 13 2021

- ROM Image Checksum: baf92c8f

- ME Firmware: 15.0.10.1469

- EC Firmware: TIGA360000

- PMC Firmware: 150.01.20.1028

- Boot Guard ACM: 1.14.15

- Memory Reference Code: Based on 0A.00.38.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.5.5115

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_00000072.pdb

New Fixes/Features:

- Based on TN0055

- [AMI Changes][EIP595175] There is a white space character when pressing Enter Key in the Enter Admin Password input window.

- [AMI Changes][EIP598186] The “Wake System from S5 (Daily/weekly ..etc)” Help Text was missing after loading F9 Defaults.

- [AMI Changes][EIP605593] WMI SPEC 6.2.3 method 601 WDT & 602 function 3,4,5,6 global reset cause

- [ODM Changes][EIPnone] Updated: EC FW to 360000

Known Errata:

- Flashing from TN0052 or TN0053 to TN0054 will still stay on POST splash screen logo for about 4mins, TN0054 to TN0054 will not see this 4 minute wait.

- TN0032 EIP560766 Added code change for new DmiEdit tool, please use new tools package from May 28, 2021 or later.

- EIP530307, With Intel comments that SAGV is not functional at this moment, therefore it is disabled in current TN BIOS.

- WU files for test use only.

### TNTGL357.0055.2021.0323.1423 Development BIOS

About This Release:

- Date: Mar, 23 2021

- ROM Image Checksum: a358638d

- ME Firmware: 15.0.10.1469

- EC Firmware: TIGA350000

- PMC Firmware: 150.01.20.1028

- Boot Guard ACM: 1.14.15

- Memory Reference Code: Based on 0A.00.38.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.5.5115

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_00000072.pdb

New Fixes/Features:

- Based on TN0054

- [AMI Changes][EIP562233] After re-plugging HDMI/Type-C cable on the boot menu screen, the screen is abnormal.

- [AMI Changes][EIP594087] Intel PETs - System cannot pass AMT_082 test item patch for SOL.

- [AMI Changes][EIP600112] OFBD module SMI handler vulnerabilities.

- [AMI Changes][EIP601899] Fix when Fan Control mode=Custom, CPU fan is not spinning.

- [ODM Changes][EIPnone] Updated: EC FW 350000

Known Errata:

- Flash from TN0052 or TN0053 to TN0054 still stay on POST logo for about 4mins, TN0054 to TN0054 will not wait 4 mins.

- TN0032 EIP560766 Add code change for new DmiEdit tool, please use new tools

- EIP530307, With Intel comment that SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- WU files for test use only.

Tiger Lake-U Standard BIOS

PRODUCTS: NUC11TNBi3/K/H, NUC11TNBi5/K/H, NUC11TNBi7/K/H (Tiger Canyon)

### TNTGL357.0054.2021.0316.1417 Production BIOS

About This Release:

- Date: Mar, 16 2021

- ROM Image Checksum: 934ba40c

- ME Firmware: 15.0.10.1469

- EC Firmware: TIGA340000

- PMC Firmware: 150.01.20.1028

- Boot Guard ACM: 1.14.15

- Memory Reference Code: Based on 0A.00.38.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.5.5115

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_00000072.pdb

New Fixes/Features:

- Based on TN0053

- [AMI Changes][EIP594087] Update Secure Erase related module for AMT_084.

- [AMI Changes][EIP595787] Fix "Disable Persistent Display Emulation" option hanging issue.

- [AMI Changes][EIP600112] OFBD module SMI handler vulnerabilities.

- [AMI Changes][EIP600410] Fix system will stay on POST logo screen for about 4 minutes before flash update starts.

Known Errata:

- BIOS flashing from TN0052 / TN0053 to TN0054 still stay on POST logo screen for about 4 minutes during flash process. Flashing TN0054 to TN0054 will not see this delay.

- TN0032 EIP560766 Added code change for new DmiEdit tool, please use new tools.

- EIP530307, With Intel comment that SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- WU files for test use only.

### TNTGL357.0053.2021.0309.1101 Development BIOS

About This Release:

- Date: Mar, 9 2021

- ROM Image Checksum: 22dddddc

- ME Firmware: 15.0.10.1469

- EC Firmware: TIGA340000

- PMC Firmware: 150.01.20.1028

- Boot Guard ACM: 1.14.15

- Memory Reference Code: Based on 0A.00.38.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.5.5115

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_00000072.pdb

New Fixes/Features:

- Based on TN0052

- [AMI Changes][EIP594087]Added AMT_084 fail on AMI BIOS patch 2.

- [AMI Changes][EIP597866]Update NTFS_06 for "Heap corruption and denial of service in NTFS DXE driver in process of parsing NTFS file system partition".

- [AMI Changes][EIP598439]Persistent display emulation miss post info after HDMI re-plug in.

Known Errata:

- TN0032 EIP560766 Add code change for new DmiEdit tool, please use new tools

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

Tiger Lake-U Standard BIOS

PRODUCTS: NUC11TNBi3/K/H, NUC11TNBi5/K/H, NUC11TNBi7/K/H (Tiger Canyon)

### TNTGL357.0052.2021.0223.1437 Production BIOS

About This Release:

- Date: Feb, 23 2021

- ROM Image Checksum: 8a24d41c

- ME Firmware: 15.0.10.1469

- EC Firmware: TIGA340000

- PMC Firmware: 150.01.20.1028

- Boot Guard ACM: 1.14.15

- Memory Reference Code: Based on 0A.00.38.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.5.5115

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_00000072.pdb

New Fixes/Features:

- Based on TN0051

- [AMI Changes][EIP595787] Persistent display emulation miss post info after HDMI re-plug in .

Known Errata:

- TN0032 EIP560766 Added code change for new DmiEdit tool, please use new tools.

- EIP530307, With Intel comment that SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- WU files for test use only.

### TNTGL357.0051.2021.0218.1210 Development BIOS

About This Release:

- Date: Feb, 18 2021

- ROM Image Checksum: b5aa5793

- ME Firmware: 15.0.10.1469

- EC Firmware: TIGA340000

- PMC Firmware: 150.01.20.1028

- Boot Guard ACM: 1.14.15

- Memory Reference Code: Based on 0A.00.38.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.5.5115

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_00000072.pdb

New Fixes/Features:

- Based on TN0050

- [AMI Changes][EIP586193] System hang when retimer update thru type-C

- [AMI Changes][EIP594087] AMT_081~AMT086 Fail after change Secure Erase mode to "Real" Mode.

- [AMI Changes][EIP595807] Retimer dynamic update function

- [ODM Changes][EIPnone] Update EC FW TIGA340000

- [ODM Changes][EIPnone] Update retimer FW 2.17 signed

Known Errata:

- TN0032 EIP560766 Added code change for new DmiEdit tool, please use new tools

- EIP530307, With Intel comment that SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- WU files for test use only.

### TNTGL357.0050.2021.0205.1744 Development BIOS

About This Release:

- Date: Feb, 5 2021

- ROM Image Checksum: 6b259c8a

- ME Firmware: 15.0.10.1469

- EC Firmware: TIGA330000

- PMC Firmware: 150.01.20.1028

- Boot Guard ACM: 1.14.15

- Memory Reference Code: Based on 0A.00.38.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.5.5115

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_00000072.pdb

New Fixes/Features:

- Based on TN0048

- [AMI Changes][EIP591437] Change virtual display setup item.

- [AMI Changes][EIP591998] ME upgrade fail by security jumper recovery.

- [AMI Changes][EIP593188] Configure Secure Erase mode to Real mode for vPro.

- [AMI Changes][EIP594079] TXT update to 1.14.15

- [ODM Changes][EIPnone] Update EC FW TIGA330000

Known Errata:

- F7 Update from TN0046 to TN0047 can update ME FW, but security jumper recovery does not update ME FW.

- TN0032 EIP560766 Added code change for new DmiEdit tool, please use new tools.

- EIP530307, With Intel comment that SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

### TNTGL357.0049.2021.0202.1138 Development BIOS

About This Release:

- Date: Feb, 2 2021

- ROM Image Checksum: 8b675526

- ME Firmware: 15.0.10.1414

- EC Firmware: TIGA320000

- PMC Firmware: 150.01.20.1026

- Boot Guard ACM: 1.14.12

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_0000006C.pdb

New Fixes/Features:

- Based on TN0043

- [AMI Changes][EIP585831]USB2.0 port has no function after disabling Thunderbolt™ Support in the BIOS.

- [AMI Changes][EIP585888]Change Cooling-Balanced Fan Off Temperature of CPUVR default.

- [AMI Changes][EIP586779]Changed L10 SKU number for energy star.

- [AMI Changes][EIP587756]IGD Primary Video Port disable/hidden not support item.

- [AMI Changes][EIP590745]enable "Enable TCO Timer" setup option.

- [AMI Changes][EIP591998]ME upgrade fail by security jumper recovery.

- [AMI Changes][EIPnone]Add SpgSelfHealing module.

- [AMI Changes][EIPnone]Fix CEC feature not working.

- [AMI Changes][EIPnone]Update ME_FW_IMAGE_VERSION token.

Known Errata:

- TN0046 to TN0048 security jumper recovery will need to recovery twice to update ME FW.

- F7 Update from TN0046 to TN0047 can update ME FW, but security jumper recovery does not update ME FW.

- TN0032 EIP560766 Added code change for new DmiEdit tool, please use new tools.

- EIP530307, With Intel comment that SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

### TNTGL357.0048.2021.0126.1444 Development BIOS

About This Release:

- Date: Jan, 26 2021

- ROM Image Checksum: 416effc9

- ME Firmware: 15.0.10.1469

- EC Firmware: TIGA320000

- PMC Firmware: 150.01.20.1028

- Boot Guard ACM: 1.14.14

- Memory Reference Code: Based on 0A.00.38.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.5.5115

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_00000072.pdb

New Fixes/Features:

- [AMI Changes][EIP591998] ME upgrade fail by security jumper recovery.

- [AMI Changes][EIPnone] Adjust POST screen QRCode position.

Known Errata:

- TN0046 to TN0048 security jumper recovery will need to recovery twice to update ME FW.

- F7 Update from TN0046 to TN0047 can update ME FW, but security jumper recovery does not update ME FW.

- TN0032 EIP560766 Added code change for new DmiEdit tool, please use new tools.

- EIP530307, With Intel comment that SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

### TNTGL357.0047.2021.0119.1013 Development BIOS

About This Release:

- Date: Jan, 19 2021

- ROM Image Checksum: 4ead1ddf

- ME Firmware: 15.0.10.1469

- EC Firmware: TIGA320000

- PMC Firmware: 150.01.20.1028

- Boot Guard ACM: 1.14.14

- Memory Reference Code: Based on 0A.00.38.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.5.5115

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_00000072.pdb

New Fixes/Features:

- [AMI Changes][EIP589872] Implement ME FW 15.0.10.1469 in to BIOSv47.

- [AMI Changes][EIP590745] Enable "Enable TCO Timer" setup option.

- [AMI Changes][EIPnone] Update ME_FW_IMAGE_VERSION token for ME FW update problem.

Known Errata:

- F7 Update from TN0046 to TN0047 can update ME FW, but security jumper recovery does not update ME FW.

- TN0032 Re-sign BIOS Guard part failed, cannot do TN0032 to TN0032 same version capsule flash, please use FPT.

- TN0032 EIP560766 Add code change for new DmiEdit tool, please use new tools.

- Fab B MB cannot support EC v25, please keep Fab B MB EC up to v24.

- Before Flash Fab C MB BIOS make sure the EC version is after v25.

- EIP530307, With Intel comment that SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

### TNTGL357.0046.2021.0112.1533 Development BIOS

About This Release:

- Date: Jan, 12 2021

- ROM Image Checksum: 14520c8a

- ME Firmware: 15.0.10.1447

- EC Firmware: TIGA320000

- PMC Firmware: 150.01.20.1028

- Boot Guard ACM: 1.14.14

- Memory Reference Code: Based on 0A.00.38.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.5.5115

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_00000072.pdb

New Fixes/Features:

- [AMI Changes][EIPnone] Added BIOS Self-Help setup item.

- [AMI Changes][EIPnone] Fix CEC feature not working.

- [AMI Changes][EIPnone] Update ESA label 053.

- [ODM Changes][EIPnone] Update GOP VBT Tiger_TGL_GOP_17.0.1052_20210107_HDMI-B_disable_Pcon.bin.

Known Errata:

- TN0032 Re-sign BIOS Guard part failed, cannot do TN0032 to TN0032 same version capsule flash, please use FPT.

- TN0032 EIP560766 Add code change for new DmiEdit tool, please use new tools.

- Fab B MB cannot support EC v25, please keep Fab B MB EC up to v24.

- Before Flash Fab C MB BIOS make sure the EC version is after v25.

- EIP530307, With Intel comment that SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

### TNTGL357.0045.2021.0107.1501 Development BIOS

About This Release:

- Date: Jan, 7 2021

- ROM Image Checksum: 1eb47911

- ME Firmware: 15.0.10.1447

- EC Firmware: TIGA320000

- PMC Firmware: 150.01.20.1028

- Boot Guard ACM: 1.14.14

- Memory Reference Code: Based on 0A.00.38.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1052

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.5.5115

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_00000072.pdb

New Fixes/Features:

- [AMI Changes][EIP586779] Changed L10 SKU number for energy star.

- [AMI Changes][EIP587756] IGD Primary Video Port disable/hidden not support item.

- [AMI Changes][EIP587971] Update CPU Microcode to m80806c1_00000072.

- [AMI Changes][EIPnone] Update GOP to 17.0.1052.

- [ODM Changes][EIP585831] USB2.0 port has no function after disabling Thunderbolt™ Support in the BIOS.

Known Errata:

- TN0032 Re-sign BIOS Guard part failed, cannot do TN0032 to TN0032 same version capsule flash, please use FPT.

- TN0032 EIP560766 Add code change for new DmiEdit tool, please use new tools.

- Fab B MB cannot support EC v25, please keep Fab B MB EC up to v24.

- Before Flash Fab C MB BIOS make sure the EC version is after v25.

- EIP530307, With Intel comment that SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

### TNTGL357.0044.2020.1229.1900 Development BIOS

About This Release:

- Date: Dec 29, 2020

- ROM Image Checksum: b564254c

- ME Firmware: 15.0.10.1447

- EC Firmware: TIGA320000

- PMC Firmware: 150.01.20.1028

- Boot Guard ACM: 1.14.14

- Memory Reference Code: Based on 0A.00.38.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.5.5115

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_0000006E.pdb

New Fixes/Features:

- [AMI Changes][EIP585831] USB2.0 port has no function after disabling Thunderbolt™ Support in the BIOS.

- [AMI Changes][EIP585888] Change Cooling-Balanced Fan Off Temperature of CPUVR default.

- [AMI Changes][EIPnone] Add SpgSelfHealing module.

- [AMI Changes][EIPnone] Changed L10 SKU number for energy star.

- [AMI Changes][EIPnone] Update TGL CRB Label 32 and BKC for 20H2.

Known Errata:

- TN0032 Re-sign BIOS Guard part failed, cannot do TN0032 to TN0032 same version capsule flash, please use FPT.

- TN0032 EIP560766 Added code change for new DmiEdit tool, please use new tools.

- Fab B MB cannot support EC v25, please keep Fab B MB EC up to v24.

- Before Flash Fab C MB BIOS make sure the EC version is after v25.

- EIP530307, With Intel comment that SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

Tiger Lake-U Standard BIOS

PRODUCTS: NUC11TNBi3/K/H, NUC11TNBi5/K/H, NUC11TNBi7/K/H (Tiger Canyon)

### TNTGL357.0043.2020.1223.1022 (non-vPro) Production BIOS

About This Release:

- Date: Dec 23, 2020

- ROM Image Checksum: 41AA299A

- ME Firmware: 15.0.10.1414

- EC Firmware: TIGA320000

- PMC Firmware: 150.01.20.1026

- Boot Guard ACM: 1.14.12

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_0000006C.pdb

New Fixes/Features:

- [AMI Changes][EIP554146]When disable LAN in BIOS, active/link LED is still on.

- [AMI Changes][EIP576751]Update the re-sign package (resign eip 559914 Update "TGL Resign Script 20201130" script to fix Fault Tolerance fail.)

- [AMI Changes][EIP579844][EIP579841]Fix modern standby blinking behavior

- [AMI Changes][EIPnone]Retimer FW update version control

Known Errata:

- TN0032 Resign BIOS Guard part failed, cannot do TN0032 to TN0032 same version capsule flash, please use FPT.

- TN0032 EIP560766 Add code change for new DmiEdit tool, please use new tools.

- Fab B MB cannot support EC v25, please keep Fab B MB EC up to v24.

- Before Flashing Fab C MB BIOS, make sure the EC version is newer than v25.

- EIP530307, With Intel comment that SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- WU files for test use only.

### TNTGL357.0042.2020.1221.1743 Development BIOS

About This Release:

- Date: Dec 22, 2020

- ROM Image Checksum: ab4d454f

- ME Firmware: 15.0.10.1414

- EC Firmware: TIGA320000

- PMC Firmware: 150.01.20.1026

- Boot Guard ACM: 1.14.12

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C1_0000006C.pdb

New Fixes/Features:

- [AMI Changes][EIPnone] Revert set GPP_E2 to low when DisableWOL.

- [AMI Changes][EIPnone] Remove TBT retimer FW from capsule.

Known Errata:

- TN0032 Re-sign BIOS Guard part failed, cannot do TN0032 to TN0032 same version capsule flash, please use FPT.

- TN0032 EIP560766 Added code change for new DmiEdit tool, please use new tools.

- Fab B MB cannot support EC v25, please keep Fab B MB EC up to v24.

- Before Flash Fab C MB BIOS make sure the EC version is after v25.

- EIP530307, With Intel comment that SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- WU files for test use only.

### TNTGL357.0041.2020.1215.1058	Development  BIOS

About This Release:

- Date: Dec 15, 2020

- ROM Image Checksum: 75ce2224

- ME Firmware: 15.0.10.1414

- EC Firmware: TIGA320000

- PMC Firmware: 150.01.20.1026

- Boot Guard ACM: 1.14.12

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C1_0000006C.pdb

New Fixes/Features:

- [AMI Changes][EIP579849]Fix "POST Logo" setup item function

- [AMI Changes][EIP579852]Fixed that the "Unlimited Boot to Network Attempts" item could not work correctly.

- [AMI Changes][EIP582726]Fix minimum duty cycle default not change when use capsule flash

- [AMI Changes][EIP583222]Disable COM2 in device manager

- [AMI Changes][EIPnone]Change USB tuning value 0xFDCA4500 0xFDCA4600

- [AMI Changes][EIPnone]Change dual PXE "Network Boot" setup item display behavior

- [AMI Changes][EIPnone]Change power button press 20sec not trigger RTC reset

- [AMI Changes][EIPnone]Fix modern standby LED not blinking

- [AMI Changes][EIPnone]Set GPP_E2 to low when DisableWOL

- [ODM Changes][EIPnone]Update EC FW 320000

Known Errata:

- TN0032 Resign BIOS Guard part failed, cannot do TN0032 to TN0032 same version capsule flash, please use FPT.

- TN0032 EIP560766 Add code change for new DmiEdit tool, please use new tools

- Fab B MB can not support EC v25, please keep Fab B MB EC up to v24.

- Before Flash Fab C MB BIOS make sure the EC version is after v25.

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

### TNTGL357.0040.2020.1208.1432	Development  BIOS

About This Release:

- Date: Dec 8, 2020

- ROM Image Checksum: 59fdfd03

- ME Firmware: 15.0.10.1414

- EC Firmware: TIGA310000

- PMC Firmware: 150.01.20.1026

- Boot Guard ACM: 1.14.12

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C1_0000006C.pdb

New Fixes/Features:

- [AMI Changes][EIP569412]"OK button" shows the wrong position in the Secure Boot Violation screen.

- [AMI Changes][EIP577984]PXE boot function support on dual LAN SKU

- [AMI Changes][EIP579844][EIP579844]MS indicator still blinking after setting "Modern Standby Indicator Brightness" to OFF

- [AMI Changes][EIP581580]Change USB 2.0 tunning value

- [AMI Changes][EIP582726]Cooling-Balanced Minmun Duty Cycle default should be 42

- [AMI Changes][EIPnone]"USB Provisioning of AMT" default change to disabled

- [AMI Changes][EIPnone]Enable BiosGuard, BIOS LOCK, ME Region Access Lock, Fpf HwBindingEn for vpro SKU

Known Errata:

- TN0032 Resign BIOS Guard part failed, cannot do TN0032 to TN0032 same version capsule flash, please use FPT.

- TN0032 EIP560766 Add code change for new DmiEdit tool, please use new tools

- Fab B MB can not support EC v25, please keep Fab B MB EC up to v24.

- Before Flash Fab C MB BIOS make sure the EC version is after v25.

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

### TNTGL357.0039.2020.1201.1421 Development BIOS

About This Release:

- Date: Dec 1, 2020

- ROM Image Checksum: 331c23ae

- ME Firmware: 15.0.10.1414

- EC Firmware: TIGA310000

- PMC Firmware: 150.01.20.1026

- Boot Guard ACM: 1.14.12

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C1_0000006C.pdb

New Fixes/Features:

- [AMI Changes][EIP559742]Fix WMI method 203

- [AMI Changes][EIP564316]WHQL :Secure Boot Manual Logo Test

- [AMI Changes][EIP578057][EIP580716]Fix system auto restart, if "wake on LAN from S4/S5" set to "Stay off".

- [AMI Changes][EIP578990]Fix Front Panel USB port set Disable, still show on Boot Menu.

- [AMI Changes][EIP578992]System still detect RS232 mouse after disabling "Serial port" in the BIOS.

- [AMI Changes][EIP579062]Hide "Wake from Thunderbolt" when TBT disabled

- [AMI Changes][EIP579360]Hide and force disable Audio DSP setup option

- [AMI Changes][EIP579472]System stops at post screen and cannot enter BIOS after setting rotation screen display in the BIOS

- [AMI Changes][EIP580168]IfwiRedundancyEnabled set to true

- [AMI Changes][EIP580797]Power Button Menu LED requirement to have the flashing pattern after 3 seconds.

- [AMI Changes][EIPnone]Change for Secure Flash Rom verify fail after ME 15.0.10.1447

- [AMI Changes][EIPnone]LAN & DB LAN PCIE root port 10 & 12, L1 Substates change to L1.1

- [ODM Changes][EIPnone]Update EC FW 310000

- [ODM Changes][EIPnone]Update PXE UEFI driver to ww42 E0902X7.EFI

Known Errata:

- TN0032 Resign BIOS Guard part failed, cannot do TN0032 to TN0032 same version capsule flash, please use FPT.

- TN0032 EIP560766 Add code change for new DmiEdit tool, please use new tools

- Fab B MB can not support EC v25, please keep Fab B MB EC up to v24.

- Before Flash Fab C MB BIOS make sure the EC version is after v25.

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

### TNTGL357.0038.2020.1124.1648 (non-vPro) Pilot BIOS

About This Release:

- Date: Nov 24, 2020

- ROM Image Checksum: c01834bb

- ME Firmware: 15.0.10.1414

- EC Firmware: TIGA300000

- PMC Firmware: 150.01.20.1026

- Boot Guard ACM: 1.14.12

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics:

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Visual BIOS: Intel AptioV

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .BIN & .CAP Files:

  - M80806C1_0000006C.pdb

New Fixes/Features:

- [AMI Changes][EIP545987]BIOS setting will change to default when update BIOS via CAP file.

- [AMI Changes][EIP578990]TN Rear Panel USB and  Front Panel USB port On/Off opposite issue.

- [AMI Changes][EIP578991]Fix Help Text of "Front Panel USB3 Port (Left)/Internal USB via M.2 2242 Slot Power On/Off"

- [AMI Changes][EIP579062]Add BIOS setup option "Wake from Thunderbolt" to control "VCCST states off IOM"

- [AMI Changes][EIP579474]Fix Maximum Temperature of the CPUVR secondary temperature sensor value range

- [AMI Changes][EIPnone]Add ME_FW_IMAGE_VERSION token to block ME downgrade

- [AMI Changes][EIPnone]Add OEM reserved variable

- [AMI Changes][EIPnone]Update XML file for DEBUG mode BIOS

- [AMI Changes][EIPnone]Update new PL setting table for i5/i7

- [AMI Changes][EIPnone]Update new iFlashVWin64 for EBU

- [ODM Changes][EIP579057]GPIO GPD0 and GPIO GPD1 has the correct definition

- [ODM Changes][EIP579180]ACPI OEM Table ID update for both non-vPro and vPro BIOS

- [ODM Changes][EIPnone]Update EC FW to 300000

Known Errata:

- TN0032 Resign BIOS Guard part failed, cannot do TN0032 to TN0032 same version capsule flash, please use FPT.

- EIP560766 Added code change for new DmiEdit tool, current tools are unable to use included AFUMFG, iFlashV and EBU.

- Fab B MB cannot support EC v25, please keep Fab B MB EC up to v24.

- Before Flash Fab C MB BIOS make sure the EC version is after v25.

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- WU files for test use only.

### TNTGL357.0037.2020.1118.1226	Development  BIOS

About This Release:

- Date: Nov 18, 2020

- ROM Image Checksum: c4e24a22

- ME Firmware: 15.0.10.1414

- EC Firmware: TIGA290000

- PMC Firmware: 150.01.20.1026

- Boot Guard ACM: 1.14.12

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C1_0000006C.pdb

New Fixes/Features:

- [AMI Changes][EIPnone]Update FIT setting to EomFirstBootEnabled value="Yes"

- [AMI Changes][EIPnone]Update iFlashV to 5.13.00.1990

Known Errata:

- TN0032 Resign BIOS Guard part failed, cannot do TN0032 to TN0032 same version capsule flash, please use FPT.

- EIP560766 Add code change for new DmiEdit tool, current tools is unable to use include AFUMFG, iFlashV and EBU

- Fab B MB can not support EC v25, please keep Fab B MB EC up to v24.

- Before Flash Fab C MB BIOS make sure the EC version is after v25.

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

### TNTGL357.0036.2020.1117.1528	Development  BIOS

About This Release:

- Date: Nov 17, 2020

- ROM Image Checksum: a33e25eb

- ME Firmware: 15.0.10.1414

- EC Firmware: TIGA290000

- PMC Firmware: 150.01.20.1026

- Boot Guard ACM: 1.14.12

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C1_0000006C.pdb

New Fixes/Features:

- [AMI Changes][EIP577854]Fix System Audio - UAA Test (Manual) - Desktop-Fail

- [AMI Changes][EIPnone]Change CWDT ASL code

- [AMI Changes][EIPnone]Deep S4/S5 default change to 0

- [AMI Changes][EIPnone]Enable BIOS LOCK, ME Region Access Lock, Fpf HwBindingEn for non-vpro BIOS

- [AMI Changes][EIPnone]Intel Display Audio Controller ACPI device change _ADR with 0x00000000 for modern standby

- [AMI Changes][EIPnone]vpro PttSupportedFpf=No

- [ODM Changes][EIP]Update EC FW 290000

Known Errata:

- TN0032 Resign BIOS Guard part failed, cannot do TN0032 to TN0032 same version capsule flash, please use FPT.

- EIP560766 Add code change for new DmiEdit tool, current tools is unable to use include AFUMFG, iFlashV and EBU

- Fab B MB can not support EC v25, please keep Fab B MB EC up to v24.

- Before Flash Fab C MB BIOS make sure the EC version is after v25.

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

### TNTGL357.0035.2020.1111.2245	Development  BIOS

About This Release:

- Date: Nov 11, 2020

- ROM Image Checksum: f414224e

- ME Firmware: 15.0.10.1414

- EC Firmware: TIGA280000

- PMC Firmware: 150.01.20.1026

- Boot Guard ACM: 1.14.12

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C1_0000006C.pdb

New Fixes/Features:

- [AMI Changes][EIP576910]Change PL setting detect from Board ID to CPU ID.

- [AMI Changes][EIPnone]Change PL setting table for i3

- [AMI Changes][EIP560804]DisableWDTWhenTxtCountPresent

- [AMI Changes][EIP577439]Rtd3Bt default set to disable

- [AMI Changes][EIPnone]Update BKC WW45

- [AMI Changes][EIPnone]Update microcode to 6c

Known Errata:

- TN0032 Resign BIOS Guard part failed, cannot do TN0032 to TN0032 same version capsule flash, please use FPT.

- EIP560766 Add code change for new DmiEdit tool, current tools is unable to use include AFUMFG, iFlashV and EBU

- Fab B MB can not support EC v25, please keep Fab B MB EC up to v24.

- Before Flash Fab C MB BIOS make sure the EC version is after v25.

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

### TNTGL357.0034.2020.1109.2110	Development  BIOS

About This Release:

- Date: Nov 9, 2020

- ROM Image Checksum: 5554428c

- ME Firmware: 15.0.10.1414

- EC Firmware: TIGA280000

- PMC Firmware: 150.01.20.1026

- Boot Guard ACM: 1.14.12

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C1_00000068.pdb

New Fixes/Features:

- [AMI Changes][EIP576151]The function of Rear Panel USB2 port (Bottom) is opposite to Rear Panel USB3 port (Top) in the BIOS.

- [AMI Changes][EIP576910]Change PL setting detect from Board ID to CPU ID.

- [AMI Changes][EIPnone]Disable dgr when debug mode on

- [AMI Changes][EIPnone]EC setting for power sense 5A0h and RTC reset 5A1h

- [AMI Changes][EIPnone]Fix PeiSaPolicyUpdate returns if no PEI Logo file

- [AMI Changes][EIPnone]Fix recovery jumper function

- [ODM Changes][EIP577069]implement TXT enable FITC table in to BIOS v34.

- [ODM Changes][EIPnone]Update EC FW 280000

Known Errata:

- TN0032 Resign BIOS Guard part failed, cannot do TN0032 to TN0032 same version capsule flash, please use FPT.

- EIP560766 Add code change for new DmiEdit tool, current tools is unable to use include AFUMFG, iFlashV and EBU

- Fab B MB can not support EC v25, please keep Fab B MB EC up to v24.

- Before Flash Fab C MB BIOS make sure the EC version is after v25.

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

### TNTGL357.0033.2020.1105.1840	Development  BIOS

About This Release:

- Date: Nov 5, 2020

- ROM Image Checksum: 5979af33

- ME Firmware: 15.0.10.1414

- EC Firmware: TIGA270000

- PMC Firmware: 150.01.20.1026

- Boot Guard ACM: 1.14.12

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C1_00000068.pdb

New Fixes/Features:

- [AMI Changes][EIP563488]Fix RTC reset when ECRecoveryFlag=03h enter recovery mode.

- [AMI Changes][EIP566792]Change SMBIOS_DYNAMIC_UPDATE_TPL to 16 for AMT_001 SMBIOS Table Generation fail

- [AMI Changes][EIP576116]Change Audio DSP Compliance Mode =UAA(HDA Inbox/IntelSST) as default

- [AMI Changes][EIP576116]Change DP fixed connection to No restrictions, Legacy VCCIO to TX VCCIO

- [AMI Changes][EIP576116]Change GOP VBT to Tiger_TGL_GOP_17.0.1045_20201103

- [AMI Changes][EIP576116]Change GPIO GPP_C12 setting

- [AMI Changes][EIP576116]Hide Virtual Display Emulation setup item

- [AMI Changes][EIPnone]ACPI CWDT device follow CH porting ID change to INTC1028

- [AMI Changes][EIPnone]Disable BIOS Self-Healing support

- [AMI Changes][EIPnone]Disble BIOS_LOCK for FPT

- [AMI Changes][EIPnone]Fix HDD LED mode not correctly set to power state indicator

- [AMI Changes][EIPnone]Update BIOSAC/SINIT to 1.14.12

- [AMI Changes][EIPnone]Update EC FW 270000

- [AMI Changes][EIPnone]Update to TGL CRB Label_29

Known Errata:

- TN0032 Resign BIOS Guard part failed, cannot do TN0032 to TN0032 same version capsule flash, please use FPT.

- EIP560766 Add code change for new DmiEdit tool, current tools is unable to use include AFUMFG, iFlashV and EBU

- Fab B MB can not support EC v25, please keep Fab B MB EC up to v24.

- Before Flash Fab C MB BIOS make sure the EC version is after v25.

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

### TNTGL357.0032.2020.1028.1916	Development  BIOS

About This Release:

- Date: Oct 28, 2020

- ROM Image Checksum: 2c2afc95

- ME Firmware: 15.0.10.1414

- EC Firmware: TIGA260000

- PMC Firmware: 150.01.20.1026

- Boot Guard ACM: 1.14.10

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C1_00000068.pdb

New Fixes/Features:

- [AMI Changes][EIP551574]Change ASL setting for I225 and WLAN disable D3Cold support

- [AMI Changes][EIP559742]WMI method fix for 06,07,206,503,504

- [AMI Changes][EIP560766]Add code change for new DmiEdit tool

- [AMI Changes][EIP573488]Change BIOS menu SODIMM naming

- [AMI Changes][EIP573529]Enable Audio DSP option

- [AMI Changes][EIP573544]Fan off capability control code change.

- [AMI Changes][EIPnone]Enable BIOS Guard (After review resign log, resign BIOS Guard part is failed)

- [AMI Changes][EIPnone]Follow CRB setup code update FIT_TYPE_A_CMOS when TXT enabled

- [AMI Changes][EIPnone]Update SPG module

- [AMI Changes][EIPnone]Update microcode m80806c1_00000068

- [ODM Changes][EIPnone]ME FW update to 15.0.10.1414

- [ODM Changes][EIPnone]Update EC FW 260000

Known Errata:

- TN0032 Resign BIOS Guard part failed, cannot do TN0032 to TN0032 same version capsule flash, please use FPT.

- EIP560766 Add code change for new DmiEdit tool, current tools is unable to use include AFUMFG, iFlashV and EBU

- Fab B MB can not support EC v25, please keep Fab B MB EC up to v24.

- Before Flash Fab C MB BIOS make sure the EC version is after v25.

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

### TNTGL357.0031.2020.1022.1906	Development  BIOS

About This Release:

- Date: Oct 22, 2020

- ROM Image Checksum: 4ce66e3e

- ME Firmware: 15.0.2.1377v2

- EC Firmware: TIGA240000

- PMC Firmware: 150.01.20.1024

- Boot Guard ACM: 1.14.10

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000068.pdb

  - M80806C1_00000060.pdb

New Fixes/Features:

- [AMI Changes][EIP551574]The idle resiliency is data missing on pre fab-B board

- [AMI Changes][EIP563730]Display Emulation Alert and Block Boot not expect behavior.

- [AMI Changes][EIP572384]Remove unsupported item in IgdDvmt50PreAlloc and ApertureSize

- [AMI Changes][EIPnone]Add string token for iSetupCfg

- [AMI Changes][EIPnone]Check for not flash EC on non-fab C board

- [AMI Changes][EIPnone]Intel sighting 22011388539 doc ID #613584 workaround

- [AMI Changes][EIPnone]Remove serial port 2 item to match spec

Known Errata:

- Fab B MB can not support EC v25, please keep Fab B MB EC up to v24.

- Before Flash Fab C MB BIOS make sure the EC version is after v25.

- TN0028 BIOS added fmp capsule update changed flash layout cannot capsule flash from previous BIOS, please use FPT to update BIOS.

- BKC WW38 ME FW both Cons and Corp cannot downgrade to previous version.

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

### TNTGL357.0030.2020.1013.2315	Development  BIOS

About This Release:

- Date: Oct 13, 2020

- ROM Image Checksum: 31e0113c

- ME Firmware: 15.0.2.1377v2

- EC Firmware: TIGA240000

- PMC Firmware: 150.01.20.1024

- Boot Guard ACM: 1.14.10

- Memory Reference Code: Based on 0A.00.30.51

- Integrated Graphics

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000068.pdb

  - M80806C1_00000060.pdb

New Fixes/Features:

- [AMI Changes][EIP554146]When disable LAN in BIOS,active/link LED still turn on.

- [AMI Changes][EIP559742]WMI fix for LED and display emulation

- [AMI Changes][EIP570737]Disable TCSS port 0 and 3 for TBT APP display.

- [AMI Changes][EIPnone]Change new NUC logo

- [AMI Changes][EIPnone]Update to TGL CRB Label_28

Known Errata:

- TN0028 BIOS added fmp capsule update changed flash layout cannot capsule flash from previous BIOS, please use FPT to update BIOS.

- BKC WW38 ME FW both Cons and Corp cannot downgrade to previous version.

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

### TNTGL357.0029.2020.1007.1548	Development  BIOS

About This Release:

- Date: Oct 7, 2020

- ROM Image Checksum: a67bca36

- ME Firmware: 15.0.2.1377v2

- EC Firmware: TIGA240000

- PMC Firmware: 150.01.20.1024

- Boot Guard ACM: 1.14.10

- Memory Reference Code: Based on 0A.00.2F.30

- Integrated Graphics

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000068.pdb

  - M80806C1_00000060.pdb

New Fixes/Features:

- [AMI Changes][EIP567287]Disable SaGv as workaround for memory hanging issue

- [AMI Changes][EIP569471]"Button LED" BIOS setup control LED hehavior code change.

- [AMI Changes][EIPnone]Change ACPI CWDT device and WDT_Support_Capability_AccessType

- [ODM Changes][EIPnone]Change GPIO GPP_A13, GPP_B15 setting

Known Errata:

- TN0028 BIOS added fmp capsule update changed flash layout cannot capsule flash from previous BIOS, please use FPT to update BIOS.

- BKC WW38 ME FW both Cons and Corp cannot downgrade to previous version.

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

### TNTGL357.0028.2020.0928.1754	Development  BIOS

About This Release:

- Date: Sep 28, 2020

- ROM Image Checksum: 9fa615f3

- ME Firmware: 15.0.2.1377v2

- EC Firmware: TIGA240000

- PMC Firmware: 150.01.20.1024

- Boot Guard ACM: 1.14.10

- Memory Reference Code: Based on 0A.00.2F.30

- Integrated Graphics

  - UEFI Driver: 17.0.1045

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000068.pdb

  - M80806C1_00000060.pdb

New Fixes/Features:

- [AMI Changes][EIP533621]Disable D3 support for NVME

- [AMI Changes][EIP559742]Change WMI Method 501 for Virtual Display+Persistent Display state

- [AMI Changes][EIP563725]Memory Profiles is grayed-out in the BIOS.

- [AMI Changes][EIP565412]Security hole in smbios

- [AMI Changes][EIPnone]Add Retimer FMP Capsule update function

- [AMI Changes][EIPnone]Add maximum temperature setup item

- [AMI Changes][EIPnone]Update to TGL CRB Label_27

- [ODM Changes][EIPnone]Update BKC WW38 Cons

- [ODM Changes][EIPnone]Update BKC WW38 Corp

- [ODM Changes][EIPnone]Update EC FW to 240000

Known Errata:

- TN0028 BIOS added fmp capsule update changed flash layout cannot capsule flash from previous BIOS, please use FPT to update BIOS.

- BKC WW38 ME FW both Cons and Corp cannot downgrade to previous version.

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

### TNTGL357.0027.2020.0923.1810	Development  BIOS

About This Release:

- Date: Sep 23, 2020

- ROM Image Checksum: a5f91270

- ME Firmware: 15.0.1.1345v4.2

- EC Firmware: TIGA230000

- PMC Firmware: 150.01.20.1022

- Boot Guard ACM: 1.14.10

- Memory Reference Code: Based on 0A.00.2D.72

- Integrated Graphics

  - UEFI Driver: 17.0.1044

- Intel RST Pre-OS:

  - VMD UEFI Driver: 18.0.2.5008

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000068.pdb

  - M80806C1_0000005A.pdb

New Fixes/Features:

- [AMI Changes][EIP561835][EIP563048]WHQL Modern Standby Check Thermal Zones

- [AMI Changes][EIP561893][EIP563053]WHQL: Single Computer Display Object test

- [AMI Changes][EIP562300]SMRAM overwrite with arbitrary data (microcodeUpdate)

- [AMI Changes][EIP563727]There is no option to configure "Button LED" function in the BIOS.

- [AMI Changes][EIPnone]Update microcode 5A

- [AMI Changes][EIPnone]Update to TGL CRB Label_26

- [ODM Changes][EIPnone]Change PSYS Slope default value for R1.1 board

- [ODM Changes][EIPnone]Update EC FW 230000

- [ODM Changes][EIPnone]Update GOP to 17.0.1044

- [ODM Changes][EIPnone]Update fan control table

Known Errata:

- TN0026 BIOS signed with Intel key, cannot use capsule flash from/to previous version.

- Cons ME FW cannot downgrade from 15.0.1.xxxx to 15.0.0.xxxx, Corp ME FW cannot upgrade or downgrade from 15.0.10.xxxx to 15.0.0.xxxx

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

### TNTGL357.0026.2020.0915.2200	Development  BIOS

About This Release:

- Date: Sep 15, 2020

- ROM Image Checksum: 75019cd5

- ME Firmware: 15.0.1.1345v4.2

- EC Firmware: TIGA220000

- PMC Firmware: 150.01.20.1022

- Boot Guard ACM: 1.14.10

- Memory Reference Code: Based on 0A.00.2A.30

- Integrated Graphics

  - UEFI Driver: 17.0.1042

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4897

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000068.pdb

  - M80806C1_00000052.pdb

New Fixes/Features:

- [AMI Changes][EIP544648]HDMI CEC control no function

- [AMI Changes][EIP559742]WMI method 004 006 302 value fix

- [AMI Changes][EIP561891]WHQL(Non-Vpro):Secure Boot Test - Customized Deployment Basic Test

- [AMI Changes][EIP561918]WHQL(Non-Vpro):USB Exposed Port System Test

- [AMI Changes][EIP561925]WHQL(Non-Vpro):USB Type-C ACPI Validation

- [AMI Changes][EIP562440]Implement new Display Emulation Spec setup item

- [AMI Changes][EIP563052]WHQL(Vpro):Secure Boot Test - Customized Deployment Basic Test

- [AMI Changes][EIP563054]WHQL(Vpro):USB Exposed Port System Test

- [AMI Changes][EIP563056]WHQL(Vpro):USB Type-C ACPI Validation

- [AMI Changes][EIP563413]After v21 without display MFG string during POST screen.

- [AMI Changes][EIP563722]System shows windows logo when "Fast boot" is enabled

- [AMI Changes][EIP563730]Display Emulation Alert and Block Boot not expect behavior.

- [AMI Changes][EIP563731]System can't perform BIOS Recovery via NVMe SSD.

- [AMI Changes][EIP565148]The system no BT device, when update BIOS to TN0024.

- [AMI Changes][EIPnone]Update BKC WW36 Cons, WW37 Corp

- [ODM Changes][EIPnone]Update EC FW to 220000

Known Errata:

- TN0026 BIOS signed with Intel key, cannot use capsule flash from/to previous version.

- Cons ME FW cannot downgrade from 15.0.1.xxxx to 15.0.0.xxxx, Corp ME FW cannot upgrade or downgrade from 15.0.10.xxxx to 15.0.0.xxxx

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

### TNTGL357.0025.2020.0909.1443	Development  BIOS

About This Release:

- Date: Sep 9, 2020

- ROM Image Checksum: 81ba6753

- ME Firmware: 15.0.0.1320

- EC Firmware: TIGA210000

- PMC Firmware: 150.01.20.1020

- Boot Guard ACM: 1.14.10

- Memory Reference Code: Based on 0A.00.2A.30

- Integrated Graphics

  - UEFI Driver: 17.0.1042

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4897

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000068.pdb

  - M80806C1_00000052.pdb

New Fixes/Features:

- [AMI Changes][EIP554146]Change Secondary LAN power control for new DB card

- [AMI Changes][EIP559742]Change WMI LED method 004 006

- [AMI Changes][EIP563292]Fixed DMI info write in "ODM String(Type11)String"

- [AMI Changes][EIP563728]Change default setting for "Power Sense" to enabled

- [AMI Changes][EIP563730]Fix Block Boot and Countdown are opposite.

- [AMI Changes][EIP564719]TN DBIOS request update to TXT 1.14.10

- [AMI Changes][EIPnone]Fix cannot enter internal shell

- [AMI Changes][EIPnone]The Help Text of "Secure Boot" is wrong.

- [AMI Changes][EIPnone]Update to TGL CRB Label_25

- [ODM Changes][EIPnone]ODM Request BIOS code change - remove BIOS code work around.

- [ODM Changes][EIPnone]Update EC FW 210000

- [ODM Changes][EIPnone]Update FIT XML CSME_QS_15.0.0.1320_vpro_0x62_1MHz_0908.xml

Known Errata:

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

### TNTGL357.0024.2020.0901.1615	Development  BIOS

About This Release:

- Date: Sep 1, 2020

- ROM Image Checksum: 0xA8301163

- ME Firmware: 15.0.0.1320

- EC Firmware: TIGA200000

- PMC Firmware: 150.01.20.1020

- Boot Guard ACM: 1.14.8

- Memory Reference Code: Based on 0A.00.32.2A

- Integrated Graphics

  - UEFI Driver: 17.0.1042

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4897

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000068.pdb

  - M80806C1_00000052.pdb

New Fixes/Features:

- [AMI Changes][EIP544648]HDMI CEC control no function.

- [AMI Changes][EIP551895]SUT can not boot to PXE, if "wake on LAN from s4/s5" set to "Power On-PXE Boot".

- [AMI Changes][EIP554542]Fix i3 default value for PL/PMax/Psys setting

- [AMI Changes][EIP560766]There is no "Display F8 to Activate Windows Recovery Mode" option in BIOS.

- [AMI Changes][EIP561440]Disable in the BIOS but still can be find TPM device under OS.

- [AMI Changes][EIP562228]Fix garbage in the upper left corner of post screen after system boots.

- [AMI Changes][EIPnone]Implement fan control mode

- [AMI Changes][EIPnone]Update to TGL CRB Label_23

- [ODM Changes][EIPnone]Implement EC FW 200000

Known Errata:

- TN0021 POST hotkey does not display correctly.

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

### TNTGL357.0023.2020.0825.1713	Development  BIOS

About This Release:

- Date: Aug 25, 2020

- ROM Image Checksum: 5883add4

- ME Firmware: 15.0.0.1320

- EC Firmware: TIGA190000

- PMC Firmware: 150.01.20.1020

- Boot Guard ACM: 1.14.5

- Memory Reference Code: Based on 0A.00.2A.32

- Integrated Graphics

  - UEFI Driver: 17.0.1042

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4897

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000068.pdb

  - M80806C1_00000052.pdb

New Fixes/Features:

- [AMI Changes][EIP544648]Remove HDMI3,4 option from "Display HDMI Port"

- [AMI Changes][EIP552084]Change Intel Graphics Pei Display Peim default to enable

- [AMI Changes][EIP556192]System can wake up from s4 via USB keyboard/mouse. when "USB S4/S5 Power" disable in BIOS.

- [AMI Changes][EIP558817]Add retimer FW update code change

- [AMI Changes][EIP559742]WMI integration

- [AMI Changes][EIP560804]TPM provision

- [AMI Changes][EIP560953]TCSS Port0/3 enable +TCSS check list change.

- [AMI Changes][EIP561671]Resign script support on Tiger Canyon

- [ODM Changes][EIP560016]change A23 to GPO low

- [ODM Changes][EIPnone]Implement EC FW 190000

Known Errata:

- TN0021 POST hotkey does not display correctly.

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

### TNTGL357.0022.2020.0818.1931	Development  BIOS

About This Release:

- Date: Aug 18, 2020

- ROM Image Checksum: 76f0e822

- ME Firmware: 15.0.0.1320

- EC Firmware: TIGA180000

- PMC Firmware: 150.01.20.1020

- Boot Guard ACM: 1.14.5

- Memory Reference Code: Based on 0A.00.2A.32

- Integrated Graphics

  - UEFI Driver: 17.0.1042

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4897

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000068.pdb

  - M80806C1_00000052.pdb

New Fixes/Features:

- [AMI Changes][EIP554146]When disable LAN in BIOS,active/link LED still turn on.

- [AMI Changes][EIP556188]SUT can be wake from S4/S5 when disable LAN in BIOS.

- [AMI Changes][EIP556191]Power button has no function when the action of power button is set to "Sleep", "Hibernate" or "Shut down".

- [AMI Changes][EIP559742]Update WMI porting for 0x201~203, 0x205~208, 0x301~302, 0x501~503

- [AMI Changes][EIPnone]Disable BootGuard and EC recovery for debug mode build

- [AMI Changes][EIPnone]Update ME to 15.0.0.1320v5.1 (BKC WW32 Cons/WW33 Corp)

- [AMI Changes][EIPnone]Update microcode to 52

- [ODM Changes][EIP557482]change INA223 configuration register D7~D0 to 01100000

- [ODM Changes][EIP557859]Change GPIO, VBT and DDI HPD/DDC setting for virtual display

- [ODM Changes][EIPnone]Implement EC FW 180000

Known Errata:

- TN0022 uses ME 15.0.0.1320v5.1 cannot update/downgrade from/to older version, have mentioned in ME 15.0.0.1320v5.1 Release Note chapter 3.1. Please use FPT to update BIOS, and use EC flash tool to update EC.

- TN0021 Enable BootGuard, cannot use capsule flash from/to previous version. Please use FPT to update BIOS, and use EC flash tool to update EC

- EC 170000 enabled Auto RTC Reset feature, please downgrade EC to 160000 or lower before downgrade to BIOS older than TN0021.

- TN0021 POST hotkey does not display correctly.

- TN0020 uses ME 15.0.0.1318(Cons) / 15.0.0.1305(Corp) cannot update/downgrade from TN0016 ME 15.0.0.1282, have mentioned in  ME 15.0.0.1318 Release Notes. Please use FPT to update BIOS, and use EC flash tool to update EC

- Due to update to CRB21 will change ROM layout, please use FPT to flash TN0016

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- For EIP545135, HDMI/DisplayPort Audio setup item change to HD Audio, using capsule update or downgrade between TN0010 and TN0009 or lower version could cause system hanging, please use FPT to flash BIOS, then use normal capsule update to flash EC.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

- TN0012 update to CRB018 have ROM layout and ME setting change, can only use FPT to flash from/to previous version

### TNTGL357.0021.2020.0812.1201	Development  BIOS

About This Release:

- Date: Aug 11, 2020

- ROM Image Checksum: 69952024

- ME Firmware: 15.0.0.1318(Cons) 15.0.0.1305(Corp)

- EC Firmware: TIGA170000

- PMC Firmware: 150.01.20.1019(Cons) 150.01.20.1018(Corp)

- Boot Guard ACM: 1.14.5

- Memory Reference Code: Based on 0A.00.2A.32

- Integrated Graphics

  - UEFI Driver: 17.0.1042

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4897

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000068.pdb

  - M80806C1_0000004e.pdb (Cons)

  - M80806C1_0000004c.pdb (Corp)

New Fixes/Features:

- [AMI Changes][EIP546794]Enable BootGuard Profile5 FVME

- [AMI Changes][EIP555195]Auto RTC Reset function EC and BIOS

- [AMI Changes][EIP557621]SUT boot from PXE LAN no function, boot menu don't list LAN device.

- [AMI Changes][EIPnone]Update to TGL CRB Label_22

- [AMI Changes][EIPnone]Add workaround for TGL MRC Margin Limit Check issue

- [ODM Changes][EIP557859]Update GOP VBT file for virtual display

- [ODM Changes][EIPnone]Implement EC FW 170000

Known Errata:

- TN0021 Enable BootGuard, cannot use capsule flash from/to previous version. Please use FPT to update BIOS, and use EC flash tool to update EC

- EC 170000 enabled Auto RTC Reset feature, please downgrade EC to 160000 or lower before downgrade to BIOS older than TN0021.

- TN0021 POST hotkey does not display correctly.

- TN0020 uses ME 15.0.0.1318(Cons) / 15.0.0.1305(Corp) cannot update/downgrade from TN0016 ME 15.0.0.1282, have mentioned in  ME 15.0.0.1318 Release Notes. Please use FPT to update BIOS, and use EC flash tool to update EC

- Due to update to CRB21 will change ROM layout, please use FPT to flash TN0016

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- For EIP545135, HDMI/DisplayPort Audio setup item change to HD Audio, using capsule update or downgrade between TN0010 and TN0009 or lower version could cause system hanging, please use FPT to flash BIOS, then use normal capsule update to flash EC.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

- TN0012 update to CRB018 have ROM layout and ME setting change, can only use FPT to flash from/to previous version

### TNTGL357.0020.2020.0804.2007	Development  BIOS

About This Release:

- Date: Aug 4, 2020

- ROM Image Checksum: 83a519cf

- ME Firmware: 15.0.0.1318(Cons) 15.0.0.1305(Corp)

- EC Firmware: TIGA160000

- PMC Firmware: 150.01.20.1019(Cons) 150.01.20.1018(Corp)

- Boot Guard ACM: 1.14.5

- Memory Reference Code: Based on 0A.00.2A.40

- Integrated Graphics

  - UEFI Driver: 17.0.1042

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4897

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000068.pdb

  - M80806C1_0000004e.pdb (Cons)

  - M80806C1_0000004c.pdb (Corp)

New Fixes/Features:

- [AMI Changes][EIP551363]Change GPIO GPP_A11 and GPP_A23 setting

- [AMI Changes][EIP552084]Peim enable cause HDMI 2.0 diplay hang on "0708"

- [AMI Changes][EIP554148]DB card install detect porting change

- [AMI Changes][EIP554242]UCM BIOS porting

- [AMI Changes][EIP554542]Implement PL/PMax/Psys setting

- [AMI Changes][EIPnone]Update BKC WW30

- [ODM Changes][EIPnone]Update EC FW 160000

- [ODM Changes][EIP551959]USB OC function Porting

Known Errata:

- TN0020 uses ME 15.0.0.1318(Cons) / 15.0.0.1305(Corp) cannot update/downgrade from TN0016 ME 15.0.0.1282, have mentioned in  ME 15.0.0.1318 Release Notes. Please use FPT to update BIOS, and use EC flash tool to update EC

- Due to update to CRB21 will change ROM layout, please use FPT to flash TN0016

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- For EIP545135, HDMI/DisplayPort Audio setup item change to HD Audio, using capsule update or downgrade between TN0010 and TN0009 or lower version could cause system hanging, please use FPT to flash BIOS, then use normal capsule update to flash EC.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

- TN0012 update to CRB018 have ROM layout and ME setting change, can only use FPT to flash from/to previous version

### TNTGL357.0016.2020.0728.1809	Development  BIOS

About This Release:

- Date: Jul 28, 2020

- ROM Image Checksum: 8cead28c

- ME Firmware: 15.0.0.1240(ES2), 15.0.0.1282(B0)

- EC Firmware: TIGA150000

- PMC Firmware: 150.01.10.1031

- Boot Guard ACM: 1.14.5

- Memory Reference Code: Based on 0A.00.2A.40

- Integrated Graphics

  - UEFI Driver: 17.0.1041

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4897

- AHCI Code: Based on AHCI_24

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000068.pdb

  - M80806C1_00000044.pdb

New Fixes/Features:

- [AMI Changes][EIP554141]Update to TGL CRB Label_21

- [AMI Changes][EIP554147]Secondary LAN MAC Address shows "Not available" in Main page.

- [AMI Changes][EIP555364]Integrate ESA in TN project

- [AMI Changes][EIPnone]PEP SATA default change to D0F1

- [ODM Changes][EIP555783]Implement EC TIGA150000.

Known Errata:

- Due to update to CRB21 will change ROM layout, please use FPT to flash TN0016

- EBU/iFlashV update BIOS fail. (Vpro only)

- Tool AfuMfg/iDmiEdit fail (Vpro only)

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- For EIP545135, HDMI/DisplayPort Audio setup item change to HD Audio, using capsule update or downgrade between TN0010 and TN0009 or lower version could cause system hanging, please use FPT to flash BIOS, then use normal capsule update to flash EC.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

- TN0012 update to CRB018 have ROM layout and ME setting change, can only use FPT to flash from/to previous version

### TNTGL357.0015.2020.0721.2031	Development  BIOS

About This Release:

- Date: Jul 21, 2020

- ROM Image Checksum: 26b25c55

- ME Firmware: 15.0.0.1240(ES2), 15.0.0.1282(B0)

- EC Firmware: TIGA140000

- PMC Firmware: 150.01.10.1031

- Boot Guard ACM: 1.14.5

- Memory Reference Code: Based on 0A.00.28.42

- Integrated Graphics

  - UEFI Driver: 17.0.1041

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4897

- AHCI Code: Based on AHCI_23

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000068.pdb

  - M80806C1_00000044.pdb

New Fixes/Features:

- [AMI Changes][EIP552597]Remove SPG setup PL1/2 IA VR sync to CRB setup (Note: Only CRB PL/VR setup can use in this version)

- [AMI Changes][EIP552696]Implement Fixed Fan control mode.

- [AMI Changes][EIP554542]Implement PL/PMax/Psys setting

- [AMI Changes][EIP548609]Intel logo resolution show abnormal, when connecting the LG 27MD5KA-B monitor.

- [ODM Changes][EIP551959]USB OC function Porting

- [ODM Changes][EIP554030]Implement BPG v1.8 + BKC WW28

Known Errata:

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- For EIP545135, HDMI/DisplayPort Audio setup item change to HD Audio, using capsule update or downgrade between TN0010 and TN0009 or lower version could cause system hanging, please use FPT to flash BIOS, then use normal capsule update to flash EC.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

- TN0012 update to CRB018 have ROM layout and ME setting change, can only use FPT to flash from/to previous version

### TNTGL357.0014.2020.0714.2150	Development  BIOS

About This Release:

- Date: Jul 14, 2020

- ROM Image Checksum: 840a37d7

- ME Firmware: Consumer 15.0.0.1240

- EC Firmware: TIGA140000

- PMC Firmware: 150.01.10.1031

- Boot Guard ACM: 1.12.32

- Memory Reference Code: Based on 0A.00.28.42

- Integrated Graphics

  - UEFI Driver: 17.0.1039

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4897

- AHCI Code: Based on AHCI_23

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000068.pdb

  - M80806C1_0000003a.pdb

New Fixes/Features:

- [AMI Changes][EIP544596]The LAN can be still work, when the disabled "LAN" function in the onboard device.

- [AMI Changes][EIP545809]SPI Flash Interface test fail.

- [AMI Changes][EIP548748]Update to TGL CRB Label_19

- [AMI Changes][EIP550688]2nd Lan card BIOS setup option

- [AMI Changes][EIP551802]Core IC short term solution change BIOS P2 setting to 0.

- [AMI Changes][EIPnone]Add setup "Wake from Thunderbolt Device" item

- [AMI Changes][EIPnone]Display Emulation porting

- [AMI Changes][EIPnone]Fix MEBX prompt hotkey no function

- [AMI Changes][EIPnone]Revert B0 ME FW to 1224

- [AMI Changes][EIPnone]Setup "VR Current Limit (ICCmax)" item mapiing

- [AMI Changes][EIPnone]TXT function porting

- [AMI Changes][EIPnone]Update microcode m80806c1_0000003a

- [ODM Changes][EIP551923]Implement BPG v1.7 + ES2 BKC WW26

- [ODM Changes][EIP552989]Implement EC TIGA140000.

Known Errata:

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- For EIP545135, HDMI/DisplayPort Audio setup item change to HD Audio, using capsule update or downgrade between TN0010 and TN0009 or lower version could cause system hanging, please use FPT to flash BIOS, then use normal capsule update to flash EC.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

- TN0012 update to CRB018 have ROM layout and ME setting change, can only use FPT to flash from/to previous version

### TNTGL357.0013.2020.0707.1703	Development  BIOS

About This Release:

- Date: Jul 7, 2020

- ROM Image Checksum: 458c313d

- ME Firmware: Consumer 15.0.0.1240

- EC Firmware: TIGA130000

- PMC Firmware: 150.01.10.1031

- Boot Guard ACM: 1.12.32

- Memory Reference Code: Based on 0A.00.26.70

- Integrated Graphics

  - UEFI Driver: 17.0.1039

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4897

- AHCI Code: Based on AHCI_23

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000068.pdb

  - M80806C1_00000030.pdb

New Fixes/Features:

- [AMI Changes][EIP546573]SUT connect with BT Headset the Microphone without function.

- [AMI Changes][EIP550474]USB port power on / off fail.

- [AMI Changes][EIP550784]Set HDA Link, DMIC #0, DMIC #1, SNDW #1, and SNDW #2 to dsable as default.

- [AMI Changes][EIPnone]Remove OS Selection setup item

- [AMI Changes][EIPnone]Change PL setting to match PRD

- [AMI Changes][EIPnone]Change the CRB setup PEP SATA constraint to "Adapter D3"

- [AMI Changes][EIPnone]EnableC6Dram default set to enable

- [ODM Changes][EIP550358]Implement BPG v1.5 + BKC WW26

- [ODM Changes][EIP551363]Implement BPG v1.6 + BKC WW26

- [ODM Changes][EIP551501]Implement EC TIGA130000.

Known Errata:

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- For EIP545135, HDMI/DisplayPort Audio setup item change to HD Audio, using capsule update or downgrade between TN0010 and TN0009 or lower version could cause system hanging, please use FPT to flash BIOS, then use normal capsule update to flash EC.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

- TN0012 update to CRB018 have ROM layout and ME setting change, can only use FPT to flash from/to previous version

### TNTGL357.0012.2020.0630.2007	Development  BIOS

About This Release:

- Date: Jun 30, 2020

- ROM Image Checksum: 6c48aa10

- ME Firmware: Consumer 15.0.0.1219

- EC Firmware: TIGA120000

- PMC Firmware: 150.01.10.1030

- Boot Guard ACM: 1.12.32

- Memory Reference Code: Based on 0A.00.26.70

- Integrated Graphics

  - UEFI Driver: 17.0.1038

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4437

- AHCI Code: Based on AHCI_23

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000066.pdb

  - M80806C1_00000030.pdb

New Fixes/Features:

- [AMI Changes][EIP533621]PEG PEP constraint auto detect

- [AMI Changes][EIP534909]Change LED blinking behavior in modern standby

- [AMI Changes][EIP536529]Output log via COM port 1

- [AMI Changes][EIP542055]Run VTinfo fail.

- [AMI Changes][EIP544606]Disable Thunderbolt support in BIOS but it is workable under OS.

- [AMI Changes][EIP544648]Notify EC ACPI active and add Q event 11/E3

- [AMI Changes][EIP545885]Update to TGL CRB Label_18

- [AMI Changes][EIP546542]Fix M.2 SATA port disable

- [AMI Changes][EIP546611]There is no "SATA port" item in BIOS.

- [AMI Changes][EIP546906]USB3 Port Disable fail issues

- [AMI Changes][EIP547990]The system no BT device, when update BIOS to TN0010.

- [AMI Changes][EIP550117]DC Input voltage and CPU I/O voltage readwrong value.

- [AMI Changes][EIPnone]Change default value of PchHdAudioDsp and OemDigitalMicrophone item

- [AMI Changes][EIPnone]L10/L9 SKU number porting for EnergyStar logo and F8 hotkey

- [AMI Changes][EIPnone]Smbios type 8 type 41 porting

- [AMI Changes][EIPnone]TPM/PTT item porting

- [ODM Changes][EIP547886]Implement BPG v1.2

- [ODM Changes][EIP549073]Implement EC TIGA120000

- [ODM Changes][EIP549709]Implement BPG v1.4 + support B0 CPU.

Known Errata:

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- For EIP545135, HDMI/DisplayPort Audio setup item change to HD Audio, using capsule update or downgrade between TN0010 and TN0009 or lower version could cause system hanging, please use FPT to flash BIOS, then use normal capsule update to flash EC.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

- TN0012 update to CRB018 have ROM layout and ME setting change, can only use FPT to flash from/to previous version

### TNTGL357.0011.2020.0617.1143	Development  BIOS

About This Release:

- Date: Jun 17, 2020

- ROM Image Checksum: c1864bce

- ME Firmware: Consumer 15.0.0.1219

- EC Firmware: TIGA110000

- PMC Firmware: 150.01.10.1030

- Boot Guard ACM: 1.12.32

- Memory Reference Code: Based on 0A.00.23.51

- Integrated Graphics

  - UEFI Driver: 17.0.1037

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4437

- AHCI Code: Based on AHCI_23

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000066.pdb

  - M80806C1_00000020.pdb

New Fixes/Features:

- [AMI Changes][EIP533621]Fix PEP constraints setup disable item

- [AMI Changes][EIP538116]SUT can still wake on lan, if "wake on LAN from s4/s5" set to "stay off".

- [AMI Changes][EIP535321]GPIO GPP_C14,C15 does not change when plug/unplug the jumper

- [AMI Changes][EIP546275]Post screen shows "Detected BIOS Security Jumper Removed" message.

- [AMI Changes][EIP546277]System can't boot to BIOS setup, Boot menu and Update BIOS.

- [ODM Changes][EIP541826]GPP_F4,F5,F10 will change to 1.8V

- [ODM Changes][EIP547502]Implement EC TIGA110000.

- [AMI Changes][EIP543729]Disable HD audio in BIOS menu will cause TN can not power on.

- [ODM Changes][EIP547886]Update VBT Tiger_TGL_GOP_17.0.1037_20200616.bin

- [AMI Changes][EIPnone]Native ACPI OS PCIe Support default set to Enabled

Known Errata:

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- For EIP545135, HDMI/DisplayPort Audio setup item change to HD Audio, using capsule update or downgrade between TN0010 and TN0009 or lower version could cause system hanging, please use FPT to flash BIOS, then use normal capsule update to flash EC.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

### TNTGL357.0010.2020.0610.1859	Development  BIOS

About This Release:

- Date: Jun 9, 2020

- ROM Image Checksum: 4b6e534a

- ME Firmware: Consumer 15.0.0.1219

- EC Firmware: TIGA100000

- PMC Firmware: 150.01.10.1030

- Boot Guard ACM: 1.12.32

- Memory Reference Code: Based on 0A.00.23.51

- Integrated Graphics

  - UEFI Driver: 17.0.1037

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4437

- AHCI Code: Based on AHCI_23

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000066.pdb

  - M80806C1_00000020.pdb

New Fixes/Features:

- [AMI Changes][EIP544600]Disable WLAN in BIOS but it is workable under OS.

- [AMI Changes][EIP544648]HDMI CEC control no function.

- [AMI Changes][EIP544825]The UEFI shell environment isn't full screen.

- [AMI Changes][EIP544926]update RstPeim and RST VMD UEFI driver to 18.0.0.4777

- [AMI Changes][EIP545135]There is no "HDMI/DisplayPort Audio" item in Onboard tab.

- [AMI Changes][EIP545991]M.2 Slot Information still shows "Not Installed" and no Vendor/Device ID, when M.2 PCIe (SATA / NVMe) storage device is attached to the M.2 slot.

- [AMI Changes][EIP545992]There is show "Portable Device Charging Mode" option in the BIOS.

- [AMI Changes][EIP546274]Device Management shows yellow bang (Unknown device).

- [AMI Changes][EIPnone]Add OemSmbios module

- [AMI Changes][EIPnone]Change EC fan porting

- [AMI Changes][EIPnone]Disable CIR DevEnable setting

- [AMI Changes][EIPnone]OemUsbS4S5Wake default value fix

- [AMI Changes][EIPnone]PushButtonReset module hotkey setting

- [AMI Changes][EIPnone]USB port disable fix

- [ODM Changes][EIP545138]Implement BPG v1.1 and implement BKC’WW22.

- [ODM Changes][EIP546184]Implement EC TIGA100000.

Known Errata:

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

- For EIP545135, HDMI/DisplayPort Audio setup item change to HD Audio, using capsule update or downgrade between TN0010 and TN0009 or lower version could cause system hanging, please use FPT to flash BIOS, then use normal capsule update to flash EC.

- EIP545987, BIOS setting change to default when update BIOS via CAP file in development stage, will keep default value in the future.

### TNTGL357.0009.2020.0603.1148	Development  BIOS

About This Release:

- Date: Jun 3, 2020

- ROM Image Checksum: 848f7ec7

- ME Firmware: Consumer 15.0.0.1192

- EC Firmware: TIGA090000

- PMC Firmware: 150.01.10.1028

- Boot Guard ACM: 1.12.32

- Memory Reference Code: Based on 0A.00.23.51

- Integrated Graphics

  - UEFI Driver: 17.0.1037

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4437

- AHCI Code: Based on AHCI_23

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000066.pdb

  - M80806C1_00000020.pdb

New Fixes/Features:

- [AMI Changes][EIP535321]GPIO GPP_C14,C15 does not change when plug/unplug the jumper

- [AMI Changes][EIP536229]EC fw is not update, if update the BIOS via CAP file.

- [AMI Changes][EIP542760]Update to TGL CRB Label_17

- [ODM Changes][EIP543247]Implement EC TIGA080000.

- [ODM Changes][EIP544765]Implement EC TIGA090000.

- [ODM Changes][EIP543720]Follow BKC20

- [AMI Changes][EIP544483]Boot menu can not detect the bootable USB flash device after power button shutdown and reboot.

- [AMI Changes][EIP544596]The LAN can be still work, when the disabled "LAN" function in the onboard device.

- [AMI Changes][EIP544942]USB name provide for setup item 3.3.1.3 USB Port x

- [AMI Changes][EIPnone]power sense disable porting

Known Errata:

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

### TNTGL357.0008.2020.0526.1837	Development  BIOS

About This Release:

- Date: May 26, 2020

- ROM Image Checksum: 67d3eb18

- ME Firmware: Consumer 15.0.0.1192

- EC Firmware: TIGA070000

- PMC Firmware: 150.01.10.1016

- Boot Guard ACM: 1.12.32

- Memory Reference Code: Based on 0A.00.22.31

- Integrated Graphics

  - UEFI Driver: 17.0.1035

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4437

- AHCI Code: Based on AHCI_23

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000066.pdb

  - M80806C1_00000020.pdb

New Fixes/Features:

- [AMI Changes][EIP543494] System can't resume from S4  by USB keyboard & mouse

- [AMI Changes][EIP533621] Modern Standby PEP Checker

- [AMI Changes][EIP534909] MS mode cowork BIOS and EC

- [AMI Changes][EIP539165] SUT can not wake on wireless from s4 / s5.

- [ODM Changes][EIP541826] Update GPIO table_0518

- [AMI Changes][EIP542766] selftest Dump test item fail

- [AMI Changes][EIPnone] Add OemUsbPort module

- [AMI Changes][EIPnone] Add SvnToken module

- [AMI Changes][EIPnone] Cooling page porting

Known Errata:

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

### TNTGL357.0007.2020.0521.1159	Development  BIOS

About This Release:

- Date: May 21, 2020

- ROM Image Checksum: 72ab9073

- ME Firmware: Consumer 15.0.0.1192

- EC Firmware: TIGA070000

- PMC Firmware: 150.01.10.1014

- Boot Guard ACM: 1.12.32

- Memory Reference Code: Based on 0A.00.22.31

- Integrated Graphics

  - UEFI Driver: 17.0.1031

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4437

- AHCI Code: Based on AHCI_23

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_00000066.pdb

  - M80806C1_00000020.pdb

New Fixes/Features:

- [ODM Changes][EIP541690]Implement EC TIGA070000

- [AMI Changes][EIP540605]Tcss_BIOS_Checklist rev0p6

- [AMI Changes][EIP539152]Update to TGL CRB Label_16.1

- [AMI Changes][EIPnone]EC register porting for HDMI 2.1 port CEC

Known Errata:

- EIP5303ｒ07, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

### TNTGL357.0006.2020.0513.1124	Development  BIOS

About This Release:

- Date: May 13, 2020

- ROM Image Checksum: 83de337c

- ME Firmware: Consumer 15.0.0.1192

- EC Firmware: TIGA060000

- PMC Firmware: 150.01.10.1014

- Boot Guard ACM: 1.12.32

- Memory Reference Code: Based on 0A.00.12.14

- Integrated Graphics

  - UEFI Driver: 17.0.1031

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4437

- AHCI Code: Based on AHCI_23

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_0000005E.pdb

New Fixes/Features:

- [AMI Changes][EIP531788]The Bluetooth can work on the OS, if disabled Bluetooth function in BIOS

- [AMI Changes][EIP535321]Correct BIOS config jumper to 1,2 as normal mode, 2,3 as lockdown mode

- [ODM Changes][EIP537943]SATA 2242 port BIOS porting.

- [AMI Changes][EIP539017]Change ACPI_PM_PROFILE from "2 = Mobile" to "1 = Desktop"

- [ODM Changes][EIP539205]Implement BPG v0.9.

- [ODM Changes][EIP540445]Update BPG to v1.0

- [ODM Changes][EIP539587]Com2 setting

- [AMI Changes][EIP539670]ME is recovery mode not normal mode

- [ODM Changes][EIP539667]Modify setting for TN modern standby

- [ODM Changes][EIP540367]Implement EC TIGA060000

- [ODM Changes][EIP540422]BIOS setting implementation

- [AMI Changes][EIPnone]use iFlashV to replace old IntelAFU tool in BIOS release package

Known Errata:

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

### TNTGL357.0005.2020.0506.1128	Development  BIOS

About This Release:

- Date: May 6, 2020

- ROM Image Checksum: a978383b

- ME Firmware: Consumer 15.0.0.1192

- EC Firmware: TIGA050000

- PMC Firmware: 150.01.10.1014

- Boot Guard ACM: 1.12.32

- Memory Reference Code: Based on 0A.00.12.14

- Integrated Graphics

  - UEFI Driver: 17.0.1031

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4437

- AHCI Code: Based on AHCI_23

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_0000005E.pdb

New Fixes/Features:

- [AMI Changes][EIP535617]Memory Slots used shows error in Task manager.

- [AMI Changes][EIP533621]Fix PNP0501 and HDAS error in PEP checker

- [AMI Changes][EIP537557]Enable DAM for modern standby

- [AMI Changes][EIP538528]Set GPP_C5 to GPO low.

Known Errata:

- BIOS config jumper in position 1,2 will become lockdown mode, will change to 2,3

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

### TNTGL357.0004.2020.0428.1626	Development  BIOS

About This Release:

- Date: Apr 28, 2020

- ROM Image Checksum: 85beb290

- ME Firmware: Consumer 15.0.0.1192

- EC Firmware: TIGA050000

- PMC Firmware: 150.01.10.1014

- Boot Guard ACM: 1.12.32

- Memory Reference Code: Based on 0A.00.12.14

- Integrated Graphics

  - UEFI Driver: 17.0.1031

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4437

- AHCI Code: Based on AHCI_23

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_0000005E.pdb

New Fixes/Features:

- [AMI Changes][EIP534373]RS232 port setting.

- [ODM Changes][EIP535321]GPIO GPP_C14,C15 does not change when plug/unplug the jumper

- [AMI Changes][EIP536226]The system lost aduio device, if bios update to TN357_0002 / TNV57_0002.

- [AMI Changes][EIP536229]EC fw is not update, if update the BIOS via CAP file.

- [AMI Changes][EIP536239]The onboard LAN MAC Address shows "Not available" in Main page.

- [AMI Changes][EIP536370]TN USB2.0 value tunning

- [ODM Changes][EIP537142]Update BPG to v0.81

- [AMI Changes][EIP537339]Change Gfx frequency from 750MHz to Auto.

- [ODM Changes][EIP537398]Implement EC TIGA050000

- [AMI Changes][EIP536849]Update to TGL CRB Label_15 (Disable ControlIommu item for CRB15 update)

Known Errata:

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

### TNTGL357.0003.2020.0421.1515	Development  BIOS

About This Release:

- Date: Apr 21, 2020

- ROM Image Checksum: b0128596

- ME Firmware: Consumer 15.0.0.1166

- EC Firmware: TIGA040000

- PMC Firmware: 150.01.10.1014

- Boot Guard ACM: 1.12.32

- Memory Reference Code: Based on 0A.00.12.14

- Integrated Graphics

  - UEFI Driver: 17.0.1031

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4437

- AHCI Code: Based on AHCI_23

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_0000005C.pdb

New Fixes/Features:

- [AMI Changes][EIP533621] Modern Standby PEP Checker

- [AMI Changes][EIP534334] Insert USB flash in rear I/O the bottom USB port has not detected sound.

- [AMI Changes][EIP534373] RS232 port setting

- [AMI Changes][EIP534727] Power LED blinks on the s0 states.

- [AMI Changes][EIP534909] MS mode cowork BIOS and EC

- [ODM Changes][EIP535611] SATA Tune Value in BIOS

- [ODM Changes][EIP535704] Implement Formal EC TIGA040000

- [AMI Changes][EIP534293] HDMI 2.1 port no signal output.

- [AMI Changes][EIP514278] TigerLake cannot boot OS after adding SPG module

- [AMI Changes][EIP533238] Cannot use the bin file automatically generated by source code.

- [AMI Changes][EIP None] Align TGL RC behavior to modify TBT related BIOS setting on TN

- [AMI Changes][EIP531787] Remove PostMemGpioTable

Known Errata:

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

### TNTGL357.0002.2020.0414.1801	Development  BIOS

About This Release:

- Date: Apr 14, 2020

- ROM Image Checksum: bb3c9d39

- ME Firmware: Consumer 15.0.0.1166

- EC Firmware: TIGA030000

- PMC Firmware: 150.01.10.1014

- Boot Guard ACM: 1.12.32

- Memory Reference Code: Based on 0A.00.12.14

- Integrated Graphics

  - UEFI Driver: 17.0.1031

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4437

- AHCI Code: Based on AHCI_23

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_0000005C.pdb

New Fixes/Features:

- [AMI Changes][EIP531419] TN install PCIe wifi card and power on into OS will get BSOD.

- [AMI Changes][EIP533028] INA223 porting value in BIOS.

- [AMI Changes][EIP533257] BPG update to V7

- [AMI Changes][EIP533283] Set ITE register 0xFC to 0xFE.

- [AMI Changes][EIP533450] post screen should not have "EVALUATION COPY" text

- [AMI Changes][EIP534242] BKC WW14 for TCSS

- [AMI Changes][EIP534373] RS232 port setting.

- [ODM Changes][EIP534216] Implement EC TIGA030000

Known Errata:

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

### TNTGL357.0001.2020.0401.1535	Development  BIOS

About This Release:

- Date: Mar 31, 2020

- ROM Image Checksum: 9100d7bc

- ME Firmware: Consumer 15.0. 0.1166

- EC Firmware: TIGA020001

- PMC Firmware: 150.01.10.1013

- Boot Guard ACM: 1.12.32

- Memory Reference Code: Based on 0A.00.12.14

- Integrated Graphics

  - UEFI Driver: 17.0.1031

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4437

- AHCI Code: Based on AHCI_23

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_0000005C.pdb

New Fixes/Features:

- [AMI Changes][EIP529984] system can't power on when the monitor is inserted in the type-C port.

- [AMI Changes][EIP530473] Follow Doc 617016 to check BIOS code TCSS setting.

- [ODM Changes][EIP531597] Implement EC TIGA020001

- [AMI Changes][EIP531863] BPG update to V6

- [AMI Changes]Added SPG module

Known Errata:

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.

### TNTGL357.0000.2020.0324.0000 Development BIOS

About This Release:

- Date: Mar 24, 2020

- ROM Image Checksum: non-vPro:27d49d1c, vPro:9986a05a

- ME Firmware: Consumer 15.0. 0.1166

- EC Firmware: N/A

- PMC Firmware: 150.01.10.1013

- Boot Guard ACM: 1.12.32

- Memory Reference Code: Based on 0A.00.12.14

- Integrated Graphics

  - UEFI Driver: 17.0.1031

- Intel RST Pre-OS:

  - UEFI Driver: 18.0.0.4437

- AHCI Code: Based on AHCI_22

- Wired LAN Adapter:

  - UEFI Driver: 0.8.05

- Supported Flash Devices:

  - MACRONIX    MX25L25645G    32MB

- Microcode Updates included in .ROM & .BIO Files:

  - M80806C0_0000005C.pdb

New Fixes/Features:

- [AMI Changes][EIP530166] Update to TGL CRB 5.17_1AWHY_RC0A.00.12.17_014

- [AMI Changes][EIP528852] TN TCSS BIOS Configuration

- [AMI Changes][EIP528854] Change BIOS setup option default setting.

- [AMI Changes][EIP528987] Output Debug message via UART 2

- [AMI Changes][EIP529312] Link USB OC# Function

- [AMI Changes][EIP529539] Enable EC request update LPC IO port

- [AMI Changes][EIP529810] Support fpt.efi flash.

- [AMI Changes][EIP530307] Disable SAGV in the feature

Known Errata:

- EIP530307, With Intel comment that  SAGV don’t have functional at this moment, therefore disable it in current TN BIOS.

- EIP529810, Keep BIOS unlock in BIOS in NPI phase, and will be lock BIOS before Pilot build.